The role of stormwater in the urban water cycle

J B Ellis* and D M Revitt

Urban Pollution Research Centre, Middlesex University, The Burroughs, Hendon. NW4 4BT. 
London, UK.

*Corresponding author, e-mail B.Ellis@mdx.ac.uk

ABSTRACT

The potential contributions of impermeable surface water discharges to urban water resource management strategies are considered through the development and analysis of an urban water cycle (UWC) study approach.  The structure, outputs and benefits of a UWC study are described and a test application of the methodological approach is made to the Eastside development area within the city of Birmingham, UK.  Water demand and usage rates for the proposed 170ha regeneration area are quantified and potentially significant savings identified assuming re-use of stormwater runoff which would considerably facilitate a more sustainable future urban water cycle strategy.

KEYWORDS

Urban stormwater, urban water cycle (UWC) study, water demand and usage

# INTRODUCTION

Urban water cycle (UWC) studies are intended to identify tensions between development proposals and environmental requirements as a means of addressing and facilitating potential solutions in an integrated, sustainable and cost-effective manner.  The UWC approach primarily aims to confirm that the urban water cycle infrastructure can support developments identified within regional and local planning strategies at the right time and location.  In this respect, the UWC approach provides a guidance framework for identifying the potential contributions that stormwater discharges can make to other components, and to the concerns of differing stakeholder groups, within the total urban water cycle.  Such an approach not only requires effective stakeholder partnerships but also requires data sharing in terms of technical knowledge on the urban water resource cycle which embraces water supply, wastewater, surface water, flood risk and receiving waterbody quality and ecology.  The collation and sharing of this mix of public, corporate and private information and data sources presents a considerable challenge for stakeholder partnerships concerned with urban water resource planning.

An urban water cycle (UWC) study can be regarded as being:

- a methodological approach for determining what water resource infrastructure is required, as well as where and when it will be needed,

- a risk-based approach ensuring that the planning process makes best use of available environmental capacity and is adapted to environmental, technical, costing and other major local/regional constraints

- a structural framework for stakeholder engagement and collaboration,

- a process procedure whereby diverse and disparate knowledge and information is brought together to make better and more integrated risk-based decisions on the urban water environment,

- a basis for developing stormwater management plans (SWMPs) and preliminary strategic (flood and pollution) risk assessments (SRAs) as well as ensuring compliance with other regulatory requirements.

An effective UWC study acts as a vital evidence base for local/regional development plans, showing how water services and the water environment have been considered in the strategic planning process.  As such it can facilitate a water-based comparative assessment of development option designs and locations and can feed into and underpin core planning and regulatory control strategies.  This paper describes and applies the formal structure of a UWC study approach to a proposed development to illustrate the potential contributions that stormwater runoff can make to future sustainable urban water resource management.

# AN URBAN WATER CYCLE (UWC) STUDY

Carrying out an urban water cycle (UWC) study

A UWC study normally covers two survey stages comprising outline and detailed studies followed by an implementation and review stage (Figure 1).   The lower Level 1 risk assessment study should provide a strategic scoping of the nature and extent of major flood and pollution potential (particularly for zones of high risk likelihood), and their implications for any local development plans.  The main purpose and thrust of the upper Level 2 risk assessment would be to address the identification, quantification and mitigation of uncertainties associated with the flood and pollution risks in zones/areas carrying average to minimum risks as judged by the lower Level 1 analysis.  The UWC approach is context rather than system-oriented and is an adaptive rather than optimisation approach, with the emphasis being on process, uncertainties and identification of a range of potential mitigating solutions.  A major part of both outline and detailed Level 1/2 studies are concerned with the identification, collation and sharing of relevant data and information, which may be a source of stakeholder tension where sensitive commercial data is involved (Fletcher and Deletic; 2007; Ellis and Revitt, 2010).  It is also important that stakeholders recognise the various constraints that may apply to the identification of technically feasible solutions and accept “trade-offs” between competing or conflicting objectives.

# STORMWATER WITHIN A UWC STUDY FOR BIRMINGHAM EASTSIDE

## Background and context of Birmingham Eastside

The Eastside development area represents a major urban regeneration initiative within Birmingham city centre and covers an area of 170 hectares (www.sustainable-eastside.net).   The intensity of the built-up area means that there is a high impermeable surface cover which generates large volumes of surface water runoff having very short times of concentration.   The outcome of this is that the receiving River Rae channel as well as the local highway network, has been subject to severe overland flow flooding incidents in the historic past.   However, this high impervious cover, combined with steep slopes and the low hydraulic capacities of the small diameter surface water sewers (usually less than 160mm), mean that large volumes of overland flow are generated during wet weather conditions leading to intense, localised pluvial flooding over the urban surface.  Surcharging of the separate sewers in the development area is known to occur with storms exceeding the 1:5 Return Interval (RI) event (Groundwork Birmingham & Solihull, 2007).

INITIAL SCOPING BRIEF

(Provided by Local Authority/municipality or other Steering Group which sets out the need for and scope of the outline Level 1 study)

REGIONAL/STATE OR OTHER SUB-REGIONAL GOVERNMENT SPATIAL DEVELOPMENT STRATEGY

Provides evidence base for Core Strategy and Sustainability Appraisal

LOCAL DEVELOPMENT PLANS.

Sustainability of the Core Strategy

OUTLINE LEVEL 1 STUDY

Environmental constraints analysis.  Infrastructure constraints analysis. Sustainability assessment.

OUTLINE URBAN WATER  CYCLE STRATEGY

Strategic Flood & Pollution Risk Assessment (SRAs)

DETAILED LEVEL 2 STUDY

Identifies infrastructure needed. Where and when it is needed.  Assesses if the strategy is sustainable.                       Implementation and funding.

DETAILED URBAN WATER CYCLE STRATEGY

Evidence base and potential site allocation for DPD; sustainable construction DPDs; Area action plans

DEVELOPMENT PLAN DOCUMENTS (DPDs) & SITE SPECIFIC PROPOSALS

Strategy implementation; monitoring planning applications against strategy; periodic review and revisions to strategy

STRATEGY IMPLEMENTATION

Monitoring compliance with strategy.           Managing changes/revisions to strategy.

PLANNING APPLICATIONS & APPROVALS

Figure 1.   Stages in an Urban Water Cycle (UWC) Study.

The Eastside initiative intends to promote a new city quarter based on the themes of learning, technology and industrial heritage creating up to 12,000 new jobs during the lifetime of the initiative (www.birmingham.gov.uk/eastside).  In addition, it will accommodate 3500 new dwellings as well as a range of business premises, a 3.2 ha city park and other public open spaces; detail of the proposed allocation of urban activities within the development area is given in Figure 2.  This figure illustrates that highly impermeable landuse types such as commercial, office, education and retail activities dominate the development profile.   Whilst such building types can present a highly impermeable coverage, they also offer considerable potential for green roofs, roof disconnection and associated infiltration systems as well as stormwater/greywater recycling schemes.     The discrete nature of the separate development parcels within the planned regeneration area (see Table 1 for detail), and their staged delivery, mitigates against an integrated, holistic drainage infrastructure planning approach (Ellis et al., 2010).  However, it is governance mechanisms in the planning process which comprise the major barrier at the local municipal level to the achievement of strategic integrated implementation of drainage infrastructure (Ellis and Revitt, 2010b), rather than any inherent technical difficulties.

Main stormwater concerns of and constraints to the Eastside UWC study

Outline planning permission has been granted for the various mixed landuse development parcels of Birmingham Eastside and initial site clearance phases of the development have

Figure 2.  Proposed Landuse Activities for Birmingham Eastside

Table 1.    Summary of Peak Flow Data for Eastside Water Services.

| Development Parcel | Contributing Area (m2) | Peak (Average) Water Demand (l/s) | Peak Wastewater Discharge (l/s) |
| --- | --- | --- | --- |
| 1 City Park | 42900 | 22.0 | 50 |
| 2 Curzon Park | 40470 | 32.5 (5.8) | 314 |
| 3 Curzon Gateway | 16200 | 6.0 (2.1) | 120 |
| 4 VTP200 |  |  |  |
| 5 BCU | 14164 |  | 105 |
| 6 Ventureast | 52609 | 58.0 (4.5) | 401 |
| 7 Masshouse | 62483 | (3.8) | 462 |
| 8 Martineau Galleries | 54997 | (2.5) | 407 |
| 9 City Park Gate | 18939 | (6.2) | 140 |
| 10 Millenium Point | 48562 | 1.4 | 360 |
| 11 Multi-storey Carpark | 8462 | Negligble | 63 |
| 12 Warwick Bar | 18600 | 10.0 (1.0) | 104 |
| 13 Devonshire House |  |  |  |
| 14 Typhoo Wharf | 37400 | (1.1) | 277 |
| 15 UB40 | 3965 | (0.5) | 29 |
| 16 Aston Science Park | 89030 | (3.0) | 659 |
| 17 Aston University | 254450 | (3.5) |  |
| TOTAL | 77 (ha) | ~106 | 3491 |

[After: Coyne et al., 2008]

already taken place.   The £2.7 B investment in this urban regeneration programme is being driven by Birmingham City Council who have prime responsibility for local highway and surface water drainage.  Severn Trent Water holds prime responsibility for wastewater collection and conveyance, with additional core responsibilities for aspects of surface water drainage.  The Environment Agency (EA)  have held responsibility for the regulation of surface water drainage in respect of pluvial overland flooding, although this responsibility and associated duties will pass to the local authority on implementation of the forthcoming UK government Floods & Water Management Bill (Defra, 2009).  The EA also hold full responsibilities for receiving water quality in respect of both surface and groundwaters.

The primary concerns for surface water drainage infrastructure provision across Eastside include:

- water demand and usage rates and the capacity to meet development requirements

- the need for on-site attenuation, storage and/or infiltration facilities especially in respect of overland flows associated with extreme storm events

- the need to maintain and/or extend the drainage infrastructure, including retrofitting, where appropriate

- the timetable for staged and integrated planning and delivery of drainage infrastructure controls

- the operational reliability and sustainability of the drainage network resources being delivered.

Stage 1 of the UWC study would essentially be a desktop study exercise drawing on existing technical work and strategic planning documentation provided by the local authorities and their statutory partners, identifying the constraints to and opportunities for future development growth.  In addition, the desktop study would take cognisance of strategic regional and national guidelines for drainage infrastructure controls and management.  The Stage 1 outline scoping study would consider alternative strategic options within the context of the identified constraints as illustrated in Table 2 where the increasing intensity of shading indicates greater flood risk. This matrix approach enables the identification of relative degrees of difficulty and constraints in providing adequate water related services and infrastructure to the development area.

Table 2.   Development Constraint Matrix

| Water Resources |  | Wastewater |  | Surface Water Drainage and Pluvial Flood Risk | Fluvial Flood Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Water Resource Availability | Water Supply Network | Sewer Network | Sewage Treatment Works (STW) |  | FZ1 | FZ2 | FZ3 |
| Water resource available to meet planned developments | Existing network available with spare capacity | Existing sewer network can accommodate the proposed developments | Existing STW flow headroom can accommodate the proposed developments and there are no compliance issues | Low risk of flooding within sites or downstream | Flood Zone 1; Low probability of annual exceedance (<0.1%; <1:100) |  |  |
| Water resource available but may need new source(s) to meet developments | Existing network available with no spare capacity | Existing sewer network may need to be upgraded | Existing STW flow headroom can accommodate the proposed developments but there are compliance issues | Medium risk of flooding within sites or downstream | Flood Zone 2; Medium probability of annual exceedance (1% - 0.1%; 1:100 – 1:1000) |  |  |
| Exisrting resources not adequate to meet developments | No existing sewer network available to serve the specific development parcel(s) | Existing sewer network cannot accommodate the proposed developments | Existing STW flow headroom cannot accommodate the proposed developments | High risk of flooding on development site(s) or downstream | Flood Zone 3; High probability of annual esceedance (>1%; >1:1000) |  |  |

Building on the outcomes and findings of the Stage 1 study, a more detailed Stage 2 strategic analysis should involve further technical studies of specific issues and uncertainties in conjunction with the local development planning process to ensure integrated and timely delivery and management of water services and associated infrastructure to provide more efficient and sustainable future approaches.

## Water usage and sewer surcharging in Birmingham Eastside

A first-order inventory and benchmarking of stormwater and wastewater generation within the Eastside development area has been undertaken by Coyne et al., (2008) and which has been used as a baseline input for the Level I scoping stage.  This analysis was based on a 1:2 year return interval (RI) storm event, for which an effective runoff rate (with instantaneous peak discharge) of 26.4 mm/hour was derived by reference to local rainfall records and a daily per capita water consumption rate of 150L hd-1 d-1 was assumed.   However, it should be noted that a 5mm rainfall event in April 2000 produced a peak intensity of 113 mm/hour, although the storm only occurred over a relatively short period of time. Thus a five-fold reduction in the intensity has been applied in the modelling analysis by Coyne et al (2008).  The modelling assumes that this storm design threshold meets the Environment Agency regulatory requirements that no surface flooding equivalent to the 1:30 RI storm event should result from the predicted outflows.  The analysis also assumed a 0.9 runoff coefficient for all the development parcels, with the exception of the Green Park development parcel for which a value of 0.4 was assigned.  Maximum flows in the combined sewer system (with peak sewer flow rates limited to 2 m/s) were assumed to arrive simultaneously at sewer node points and a climate change addition of 20% - 30%, as well as an 80% limitation to runoff from each development parcel, were applied to the analysis.  These requirements might serve to add to the amount of local attenuation (and/or storage) that could be necessary for outflows generated at each site.

Table 1 shows the peak water usage rates for each of the development parcels together with their predicted peak wastewater flows for the modelled 1:2 storm event.   Table 3 shows the predicted peak flows at various nodal points in the surface water sewer network and the likelihood of surcharging for the same storm event.    The tabled data is based on information

supplied by the developers who may well be overestimating some of the pipe capacities for individual sites as the majority of the surface water sewer pipes are of small diameter (<225mm).   Irrespective of these reservations and the working assumptions of both modelling approaches, it is evident from Table 3 that pluvial surface flooding can be expected for storm events exceeding the 1:30 RI.

# STORMWATER CONTRIBUTIONS TO THE URBAN WATER CYCLE IN BIRMINGHAM EASTSIDE

The two modelling approaches described above imply that anything between 7560 – 58680 m3 of instantaneous temporary attenuation storage would be required for extreme event flow control assuming that all impermeable surface runoff (but excluding highway drainage) occurs to the separate sewers.  Based on the national guidance contained in the UK government strategy document for future water usage (Defra, 2008), it could be expected that at least a 20% reduction on these estimated discharges might be achieved through water efficient fittings to WC, washbasin and dishwashers, which might reduce overall water demand to about 80 l/hd/day.  This is a very low daily consumption rate and may well be overly optimistic given the lack of “take-up” in water efficiency appliances within current (and approved) development proposals.  However, substantial reductions in storage requirements in the order of 10% - 20%, could be achieved through the introduction of green roofs and limited source infiltration controls, both of which could be readily included into the drainage design of the retail/commercial premises which feature prominently in the regeneration proposals.  The introduction of recycled “greywater” facilities could undoubtedly lead to further substantial reductions in the combined sewer discharges to treatment as reported in Table 4, which would be in the order of 35% - 40% according to the household consumption data reported by Defra (2008).

Table 3.    Peak Flows in the Eastside Surface Water Sewer System

| Sewer Node | Contributing Parcel(s) | Pipe Capacity |  | Total Peak Flow in Pipe (m3/s) | Surcharge Potential |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum (m3/s) | Maximum (m3/s) |  |  |
| A | 1,4,6,11 | 0.9 | 1.8 | 0.5 | Unlikely |
| B | 1,2,4,5,7,8,9,10,11 | 0.9 | 1.7 | 2.3 | Probable |
| C | 1 - 12, 14, 15 | 5.1 | 5.1 | 2.8 | Unlikely |
| D | 4,11 | 0.1 | 0.1 | 0.06 | Unlikely |
| E | 1,4 - 11 | 0.9 | 1.7 | 1.9 | Probable |
| F | 6 | 0.6 | 0.6 | 0.4 | Unlikely |
| G | 7 | 0.1 | 0.1 | 0.5 | Probable |
| H | 8 | 0.6 | 0.6 | 0.4 | Unlikely |
| I | 8,9 | 0.4 | 0.4 | 0.5 | Probable |
| J | 1,4,6,10,11 | 0.9 | 1.8 | 0.8 | Unlikely |
| K | 11 | 0.1 | 0.1 | 0.1 | Unlikely |
| L | 1,2,4 – 12,14,15 | 8.3 | 8.3 | 2.7 | Unlikely |
| M | 13 | 0.08 | 0.07 | 0.04 | - |
| N | 14 | 0.1 | 0.1 | 0.3 | Probable |
| O | 1,2, 4 - 11, 15 | 0.9 | 1.7 | 2.3 | Probable |
| P |  | 0.9 | 1.8 | 0.7 | Unlikely |

[After: Coyne et al, 2008]

One estimation for a 1.1 ha development parcel of residential/apartment roofing suggested that water savings of 8,700 m3/year could be achieved through rainwater harvesting with a payback time of 3 years (Faber Maunsell, 2004). The same study also claimed a 18.6 m3/year “greywater” recycling capacity for a 350 bed hotel planned for another development parcel and having a 10.5 year payback time.   A modelling analysis for a pilot 4.5ha sub-catchment within the Eastside area indicated that the strategic placement of green roofs and porous paving could reduce total storm runoff volume by up to 32% (Viavattene et al., 2010).

Table 4 indicates the potential for the implementation of rainwater harvesting and greywater recycling schemes within Birmingham Eastside together with estimated payback times (excluding lifecycle costs).  However, such recycling schemes would appear prohibitive given the high “upfront” implementation and “follow-on” maintenance costs, as well as the possible requirement for the implementation of a third “labelled” supply system.  These considerations

Table 4.  Stormwater and Greywater Re-Use in Birmingham Eastside.

| LANDUSE SECTOR | RAINWATER HARVESTING |  | GREYWATER RECYCLING |  |
| --- | --- | --- | --- | --- |
|  | Potential | Payback Time (Years) | Potential | Payback Time (Years) |
| Single residential |  | 16 |  | 44 |
| Shared residential |  | 3.1 | XX | - |
| Public community buildings |  | 6.9 | XX | - |
| Hotels |  | 38.4 |  | 10.5 |
| Commercial office buildings |  | ? | XX | - |
| Retail buildings | See Mixed use development |  |  |  |
| Industrial buildings |  | ? | ? | ? |
| Leisure buildings |  | 6.1 |  | 7.8 |
| Public open space |  | ? | XX | - |
| Mixed use developments |  | ? |  | 4.1 |

KEY:   Potential;   High potential;  XX  Negligible potential

[After: Faber Maunsell, 2004]

mean that it is highly unlikely that greywater recycling will be introduced into the Eastside developments even for non-potable uses.  Nevertheless, irrespective of these reservations, it is acknowledged by Defra (2008) that the introduction of rainwater harvesting schemes to impermeable roof surfaces could meet a significant proportion of on-site garden/lawn watering requirements which might reduce total water demand by 5% - 10%.

# CONCLUSIONS

Although the above brief demand/usage analysis of the UWC study incorporates substantial uncertainties and data limitations, it is clear that there are opportunities for future re-distribution of water resources in the Birmingham Eastside developments.  Within future water resource re-allocations, it is also evident that stormwater contributions could be significant in facilitating a more sustainable urban water cycle strategy.   Management of surface water flows during extreme events within the development area will undoubtedly require the introduction of further attenuation/storage controls which could be used as a (re)source to meet demands elsewhere in the urban water cycle, including on-site amenity lakes/fountains, lawn/garden irrigation, groundwater recharge etc., in addition to emergency supplies for fire-fighting purposes.  The realisation of such surface water redistributions to other components of the urban water cycle is largely dependent on both the political will of the city planning authorities and on developer motivation.  There is no clear indication at the present time that the Eastside regeneration will exploit stormwater as a future resource, but forthcoming UK planning legislation and environmental regulation in relation to urban pluvial flood risks and diffuse pollution may become a driver in the reconsideration of existing thinking on this issue.

REFERENCES

Coyne, R.,Last, E., Mackay, R and Sharp, P.  2008.  Birmingham Eastside Utilities Report,  SWITCH Learning Alliance report.  www.switchurbanwater.eu.

Defra.  2008.  Future Water: The Government’s Water Strategy For England.  February 2005.  Cm 7319.   Dept. Environment, Food & Rural Affairs.  HMSO.  London. UK.

Defra.  2009.  Draft Flood and Water Management Bill.  House of Commons, Environment, Food & Rural Affairs Committee (EFRA).  HC 551-1.  HMSO, London. UK

Ellis, J.B and Revitt, D.M.  2010.  The management of urban surface water drainage in England and Wales.  Water & Environ. Journ.,  24, 1 – 8.

Faber Maunsell.  2004.  Greywater Recycling and Rainwater Harvesting at Birmingham Eastside.  February 2004.   Groundwork Birmingham & Solihull.  Birmingham, UK.

Fletcher,T and Deletic, A.  2007.  Data Requirements for Integrated Urban Water Management.  Urban Water Series, UNESCO-IHP, Paris and Taylor & Francis (Routledge Group), London, UK.

Groundwork Birmingham & Solihull.  2007.  Green Roofs for Eastside.  Report January 2007.  Hyder Consulting, Birmingham. UK.

Viavattene, C., Ellis, J.B., Revitt, D.M., Seiker, H and Peters, C.  2010.  The application of a GIS-based BMP selection tool for the evaluation of hydrologic performance and storm flow reduction.  Proc. 7th Int.Conf. Sustainable Techniques and Strategies for Urban Water Management; NOVATECH10.  28 June – 1 July 2010.  GRAIE, Insa de Lyon.  Lyon, France.

ACKNOWLEDGEMENT

This study has been carried out within the framework of the European research project SWITCH (Sustainable Urban Water Management Improves Tomorrow’s City’s Health).  SWITCH is supported by the European Commission under the 6th Framework Programme (contract number: 018530-2) and contributes to the thematic priority area of “Global Change and Ecosystems)