6. DISCUSSION OF RESULTS

# RECOMMENDATIONS FOR FUTURE WORK

“Flowcalc”, the new computer program for mass and energy balance calculations, provides the user with a number of possibilities and innovations, which could be understood from the examination of the applications described in the previous chapter. They are summarized as follows:

- Performs mass and energy balances calculations for processing flow diagrams, especially for agricultural industries, with the least number of input data and provides the user with many details, concerning the results of calculations.

- It is accompanied with a number of supporting programs, written in Microsoft Visual Basic v6.0 programming language, which provides the user with a user – friendly environment, helping him to prepare the data file, to modify data and to present the calculations results on screen. During preparation of data file, it helps the user to avoid any mistakes in the input data file, by checking the input data and presenting error messages on screen in the case of mistakes.

- It is accompanied with a couple of physical properties data bank files for both chemical compounds and agricultural products, providing also the user with the possibility to change these data or to introduce new data for new components.

- Program environment itself – Microsoft Windows – is a very user – friendly environment, enabling the user to use the calculations results in many ways and to transfer them to other windows programs (Word, Excel, Power Point etc.).

- The used programming language of the executable program – Microsoft FORTRAN v.5.1 for Windows – enables quick and accurate calculations.

- The selected processing unit types, existing as calculation subroutines (modules) in the executable program, cover the requirements of many agricultural industries and in some cases, they may be used for other types of industries. For that reason, processing units like heaters, evaporators, driers, cooling towers, separators, but also simple reactors and flash separators exist in the program. User has the ability to select also processing units for the calculation of steam, water or air requirements. Such problems exist frequently in industries processing agricultural products.

- Especially for physical properties data concerning agricultural products, the program treats these products assuming that the majority of processed agricultural products exist as water solutions, water suspensions or even independent liquids. By this way, processes where there are changes in concentrations of soluble or suspended solids (such as evaporations, dilutions etc.) are calculated easily. These cases exist frequently in units processing agricultural products.

The program has been tested for many cases, described in Chapter 5. Most of them are cases existing in industries processing agricultural products. But also there are cases for other types of industries, which have been tested too. The obtained results in all cases are satisfactory, and they are in agreement with figures existing in actual plants or described in many references.

In order to make the program more useful in the future, the writer recommends the following:

- Completion of the existing data banks with new components, either chemical compounds, or agricultural products.

- Completion of the executable program by adding also equipment design subroutines

- Modification of the program in order to be used for educational reasons too.