2. THEORETICAL METHOD

# STRUCTURE AND ORGANIZATION OF THE PROGRAM

The program is written in FORTRAN v5.1 computing language of Microsoft [23, 24, 25] for Microsoft Windows and uses the method of independent calculation subroutines to simulate a number of the most frequently used processing units mainly in plants processing agricultural products. It consists of:

- A main executive program, which controls the flow of information from and to subroutines, carries out recycling calculations and finally writes the results of calculations in a results file, in the form of ASCII characters.

- A subroutine called DATAINPUT, which reads the input data from a data file in the form of ASCII characters.

- A subroutine called STREAMARR, which selects the input information for each processing unit from a general streams data array and inserts the output information from each processing unit into the same general streams data array.

- A subroutine called CALLUNIT, which calls the proper calculation subroutine simulating a processing unit.

- A set (library) of subroutines called UNITnn, where “nn” is an integer number from 01 to 22. Each subroutine simulates a different processing unit and enables calculation of output streams data from information existing in the input stream data, while the rest required data (decision variables) for calculations, are given as input data by the user.

- Subroutines for enthalpy calculations called CALCENTH, ENTHSOL, ENTHLIQ, ENTHGAS.

- Subroutines for vapour/liquid equilibrium calculations called CALCVLE, VLMI, VLIM, VLMM.

Process flow diagram, is transformed by the user to information block flow diagram. Each block represents a separated calculation subroutine, which simulates a different processing unit or a part of it, where changes in composition, temperature or pressure take place. Lines and arrows connecting the blocks show the flow of information from one calculation subroutine to the next.

Units are taken sequentially until a recycle stream is encountered. Then the loop is torn and calculation is carried out iteratively, until satisfactory convergence is obtained. The program then reverts to sequential calculation, until a new recycle stream is encountered. Recycling calculations are carried out iteratively until satisfactory convergence is obtained. The method of direct substitution is used for recycling stream calculations, i.e. the use of the last calculated recycle stream flow rate as input stream flow rate to the first block of the recycling loop. Closure tolerance value e, is set by the user, within certain ranges.

Figures (2.1), (2.2), (2.3) and (2.4) show the general form of the computer flow diagrams of main program and subroutines DATAINPUT, STREAMARR and CALLUNIΤ.

Fig. (2.1). Flow diagram of main program.

Fig. (2.2). Flow diagram of subroutine DATAINPUT.

Fig. (2.3). Flow diagram of subroutine STREAMARR.

Fig. (2.4). Flow diagram of subroutine CALLUNIT.

2.1 REQUIRED INPUT DATA

Structure and organization of the program provides the user with the ability to obtain as much information as possible from a small number of input data. The required input data are:

- Number of components NC.

- Number of used physical properties NP.

- Number of freed streams NF.

- Number of processing units (blocks) NE.

- Array of components names COMP.

- Array of physical properties PHPR.

- Array of input stream data GNFD, which contains also all streams data obtained by calculations.

- Array of required data for each processing unit EQPM.

Processes topology is defined by general array of streams data GNFD, in combination with array of data for each processing unit EQPM. There is no need of any other array to define processes streams, since they are defined directly by their starting and destination units. There is no need also to define recycling streams, since they are defined directly from processes topology.

All required input data their arrangement and their description are presented analytically in Tables (A.1), (A.2.a), (A.2.b), (A.3), (A.4), and (A.5) of Appendix A. From their description it is clear that there are the following constraints:

- Up to 14 components is allowed to be present in the defined process.

- Up to 50 streams is allowed to be present in the information block flow diagram, from which, up to 9 is allowed to be feed streams and up to 9 to be product streams.

- Up to 10 different processing units is allowed to be present in the information block flow diagram from a set of 22 different processes unit types.

- Up to 5 input streams to each process unit represented by each block is allowed to be present.

- Up to 5 output streams from each process unit represented by each block is allowed to be present.

2.2. PROGRAM ORGANIZATION

The program is based on the presence of array GNFD, in which all required information for each stream, concerning topology, phase, temperature, enthalpy and composition are present in the form of array.

Subroutine DATAINPUT reads all required input data from an input data file. At the beginning, the only existing information about streams in array GNFD, are those of the feed streams to the system.

Starting execution, the main executive program calls subroutine STREAMARR, which selects from the existing feed streams top the system, the feed streams to the first processing unit (block) in calculation order, and arranges them in an array called FEED. Then, the main program using subroutine CALLUNIT, calls the proper calculation subroutine, simulating the process unit, represented by the process block, i.e. one of the set of subroutines UNITnn. This subroutine calculates the output stream data from the unit, using information from the input stream data and the rest data (decision variables) given as input by the user, and puts them in an array called PROD. Then subroutine STREAMARR puts the data of array PROD in array GNFD, and sets all data of arrays FEED and PROD equal to zero.

The program continues execution, selecting the input streams of the second process block and calculating its output streams. Execution continues until calculations are carried out for all processes blocks and all calculated data are registered in array GNFD. If during calculations a recycle stream is encountered from the direction of the output stream from each process block, the total flow rate of the recycle stream is registered in a new array called TOTMLOLD and calculations are carried out iteratively, for the part of the block flow diagram, which includes the recycling stream, using the method of direct substitution, i.e. the use of the last calculated recycle stream flow rate as input stream flow rate to the first block of the recycling loop. A new calculated total flow rate of the recycle stream is registered in a new array called TOTMLNEW and is compares with the corresponding value in array TOTMLOLD, according to eq. (1.3), where closure tolerance is set by the user. If satisfactory convergence is not obtained, the registered value in array TOTMLOLD is substituted by the corresponding of array TOTMLNEW and calculations are continued in the recycling loop, until satisfactory convergence is obtained. This procedure is possible to be carried out up to NCMAX times, counted and registered in array NCOUNTER.

Finally, all calculation results are written in the form of ASCII characters in a results file, which will be used for the final results presentation.

Table (A.6) of Appendix A gives a short description of the most important variables used by the program.

2.3. SUBROUTINES SIMULATING PROCESSING UNITS

Calculation subroutines (modules) existing in the program cover a wide range of actual processes. They are based on analytical methods for simulation, which are described in next chapter, while the required input data for each one, are given in Tables (A.7.01) – (A.7.22) of Appendix A.

The selection of simulated process in the program was done in such a way, to cover the requirements of industries processing agricultural products. Also they can be used to simulate simple processes for other types of industries. For that reason, the emphasized processes are heaters, evaporators, driers, cooling towers, but also simple reactors, heaters with phase change, in the process side, separators, etc. The program provides also the user with the possibility to select processes, which calculate the requirements in water, steam or air. It is important to point out that the program uses simplified calculation methods, which requires a small number of input data by the user.

2.4. USER FRIENDLY ENVIRONMENT

Microsoft Windows itself is a user-friendly environment, providing the user with a lot of Windows facilities. On the other hand, the program is running under a shell program called FLOWCALC, written in VISUAL BASIC v6.0 [26], which helps the user:

- To prepare the input data file in the form of ASCII characters, necessary for the main program to carry out the flow-sheeting calculations.

- To present the calculations results existing in an output data file in the form of ASCII characters.

- To present or modify, physical properties data existing in the internal data bank file. User can also add his data for components physical properties.

2.5. PHYSICAL PROPERTIES DATA BANK

A physical properties data bank was developed for both, chemical compounds and agricultural products, in two data files, in the form of ASCII characters: PHPRCHEM.DAT, which contains data for chemical compounds and PHPRAGRI.DAT, which contains data for agricultural products. For the case of chemical compounds, program uses the data form described in Table (A.2.a) of Appendix A. For the case of agricultural products, the program treats these products assuming that the majority of these products exist either as soluble solids or insoluble solids in water, or in some cases as liquids, where no phase change takes place during processing. By this way, processes where there are changes in concentrations of soluble or suspended solids (such as evaporations, dilutions etc.) are calculated easily. These cases exist frequently in units processing agricultural products. In that case, program uses the data form described in Table (A.2.b) of Appendix A.