3. MATHEMATICAL METHOD

# MATHEMATICAL FORMULATION OF THE PROGRAM

In this chapter, all mathematical methods used by the main executive program and by subroutines for enthalpy calculations and for vapour/liquid equilibrium calculations are described. Also, all mathematical models used by all calculation subroutines, simulating the different processing units (modules), are described.

3.1. MAIN EXECUTIVE PROGRAM

The main calculation control, which is carried out by the main executive program is that of recycling convergence, described by the following equation:

| Mr,k   - Mr,k-1 | < e                                      (3.1)

where: Mr,k , Mr,k-1 = total streams flow rates from unit r

k = number of iterations

e = required closure tolerance

The maximum number of iterations k is 10000, while the minimum required closure tolerance e is 0.0005.

3.2. SUBROUTINES FOR ENTHALPY CALCULATIONS

There are four subroutines in the program, calculating enthalpies: CALCENTH, ENTHSOL, ENTHLIQ and ENTHGAS. Enthalpy Ε of each stream is calculated with a datum temperature of Td=298°K, according to the following equation:

E  = Σnci=1  [TdTp Mi  Ci  dT ]                                 (3.2)

where: Mi = quantity of component i

Ci = specific temperature of component i

Tp = stream temperature

Td = datum temperature = 298°K

nc = total number of components

Specific heat Ci is a function of temperature T. According to Tables (A.2.a) and (A.2.b) of Appendix A it is:

- For gas phase:

Ci  = Ai + BiT + CiT²+DiT³                             (3.2.a)

- For liquid phase:

Ci = Ai + BiT                                          (3.2.b)

- For solid phase:

Ci = Ai + BiT + CiT²                                    (3.2.c)

Substituting Ci in eq. (3.2) with the corresponding function of temperature T we obtain the forms used by these subroutines, for enthalpy E calculations:

- For gas phase (from subroutine ENTHGAS):

E  = Σnci=1 Mi [Ai (Tp -Td)+Bi (Tp² - Td²)/2+Ci (Tp³ - Td³)/3+Di (Tp4 - Td4)/4]  (3.3.a)

- For liquid phase (from subroutine ENTHLIQ):

E  = Σnci=1 Mi [Ai (Tp - Td) + Bi (Tp² - Td²)/2]                    (3.3.b)

- For solid phase (from subroutine ENTHSOL):

E  = Σnci=1 Mi [Ai (Tp - Td) + Bi (Tp² - Td²)/2 + Ci (Tp³ - Td³)/3]       (3.3.c)

If there is a phase change in the interval between Td and Tp, then the proper combination of equations is selected with the addition of heat of melting and/or heat of evaporation. Subroutine CALCENTH calls the proper enthalpy calculation subroutines and controls calculations. For convenience, presentation of enthalpies equations in all calculation subroutines simulating process units, will be presented as functions of quantities, specific heats of components and temperature of stream:

Ε  = f (Mi |i=1nc , Ci |i=1nc , Tp )                                  (3.4)

3.3. SUBROUTINES FOR VAPOUR/LIQUID EQUILIBRIUM CALCULATIONS

In this program, there are four subroutines for vapour/liquid equilibrium calculations: CALCVLE, VLEMI, VLEIM and VLEMM. A list of possible combinations of liquid and gas phases, are presented in Table (3.1).

Table (3.1). List of possible combinations of liquid and gas phases for vapour/liquid equilibrium calculations. (Kern [27])

__________________________________________________________________________

- One pure liquid only.

- Completely miscible liquids only (at least 2).

- Two immiscible liquids only.

- Non-condensable gases only.

- One pure liquid with non-condensable gases.

- Completely miscible liquids (at least 2) with one immiscible liquid.

- Completely miscible liquids (at least 2) with non-condensable gases.

- Two immiscible liquids with non-condensable gases.

- Complete miscible liquids (at least 2) with one immiscible liquid and non-condensable gases.

—————————————————————————————————————————

3.3.1. Presence of completely miscible liquids only

For the case of presence of complete miscible liquids only, the total mass balance in specified temperature and pressure is (from subroutine VLEMI):

MfLtotal = Mp1total + Mp2total                               (3.5)

where: MfLtotal = total quantity

Mp1total = gas phase quantity

Mp2total = liquid phase quantity

The mass balance for each component i is:

MfLi = Mp1i + Mp2i                                     (3.6)

where: MfLi = total quantity of component i

Mp1i = quantity of component i in the gas phase

Mp2i = quantity of component i in the liquid phase

While the quantity of component i in the liquid phase is:

Mp1i = (Pvapi / Pp1) Mp1total                            (3.7)

where: Pp1 = gas phase total pressure

Pvapi = vapour pressure of component I

Combining the above equations (3.5), (3.6) and (3,7), we obtain:

MfLi

Mp1i  =   ——————————————————                (3.8)

1 + (Pp1 / Pvapi) [MfLtotal  / Mp1total - 1]

Mp2i  = MfLi - Mp1i                                       (3.9)

3.3.2. Presence of two immiscible liquids only

For the case of presence of only two immiscible liquids a and b and for the range between starting and ending of boiling (from subroutine VLEIM):

Mp1a = MfLa                                           (3.10)

Mp1b = (Pvapb / Pvapa) Mp1a                                (3.11)

where: Pvapa > Pvapb                                                                                 (3.11.a)

Mp2b = MfLb - Mp1b                                     (3.12)

3.3.3. Presence of non-condensable gases only

For the case of presence of non-condensable gases only d,e,......:

Mp1d = MfLd    ,   Mp1e = MfLe   ..........                      (3.13)

Mp2d = 0  ,   Mp2e = 0  ..........                           (3.14)

3.3.4. Combined cases

For combined cases – according to Table (3.1) – the above equations are combined accordingly (from subroutine VLEMM), while for the gas phase it should be always:

Pp1  = Σnci=1Ppari                                          (3.15)

3.4. SUBROUTINES SIMULATING PROCESSING UNITS

In this paragraph, all details concerning the used mathematical methods by each subroutine simulating processing units are described separately. All symbols used for the syntax of these equations are described in Nomenclature.

3.4.1. Mathematical model for subroutine Unit01

This type of processing unit adds (i.e. mixes) up to 5 input streams. It calculates the temperature of output stream, assuming adiabatic operation, if the temperatures of all input streams is specified, or it sets the output stream temperature equal to a specified value. It sets the output stream pressure equal to the smaller pressure of the input streams, while the phase of output stream is specified. This type of subroutine is used also to specify the temperature or the phase of a single stream.

The following equations are used:

- Phase of output stream:

Sp1=Sset                                             (3.16)

- Temperature of output stream, if it is specified:

Tp1=Tset                                             (3.17)

- Pressure of output stream:

Pp1=min(Pfj|j=1nf)                                      (3.18)

- Quantity of component i in the output stream:

Mp1i=Σnfj=1Mfji                                           (3.19)

- Enthalpy of output stream:

Ep1=Σnfj=1Efj                                            (3.20)

- Temperature of output stream in the case of adiabatic operation is calculated from:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                               (3.21)

The equation is solved using the method of iterative calculations, until satisfactory convergence is obtained. The input maximum stream temperature and the input minimum stream temperature are considered to be the upper and lower limits of Tp1, and their mean temperature is the first approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is less than the actual value of Ep1, then it is considered to be the new lower limit of Tp1, while its upper limit remains the same, and their mean value is the new approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is more than the actual value of Ep1, then it is considered to be the new upper limit of Tp1, while its lower limit remains the same, and their mean value is the new approximation of Tp1.

Iterative calculations are continued until satisfactory convergence is obtained between calculated enthalpy and actual value of Ep1, while the last value of Tp1 is the required temperature of output stream for adiabatic operation. If during iterative calculations, the difference between upper and lower value of Tp1 becomes less than 0.1°K, then their mean value is taken as the required output stream temperature Tp1.

3.4.2. Mathematical model for subroutine Unit02

This type of processing unit sets the pressure of a single stream equal to a specified value, without change in phase, temperature or composition.

The following equations are used:

- Phase of output stream:

Sp1=Sf1                                             (3.22)

- Temperature of output stream:

Tp1=Tf1                                              (3.23)

- Pressure of output stream:

Pp1=Pset                                             (3.24)

- Enthalpy of output stream:

Ep1=Ef1                                             (3.25)

- Quantity of component i in the output stream:

Mp1i=Mf1i                                           (3.26)

3.4.3. Mathematical model for subroutine Unit03

This type of processing unit represents a stream splitter, which splits a single input stream in up to 5 output streams, without change in phase, temperature, pressure or composition. The fractions of total input stream flow rate, consisting each output stream flow rate, are specified.

The following equations are used:

- Phases of each output stream k:

Spk=Sf1                                             (3.27)

- Temperatures of each output stream k:

Tpk=Tf1                                              (3.28)

- Pressures of each output stream k:

Ppk=Pf1                                             (3.29)

- Enthalpies of each output stream k:

Epk=mkEf1                                           (3.30)

- Quantities of component i in each output stream k:

Mpki=mkMf1i                                          (3.31)

3.4.4. Mathematical model for subroutine Unit04

This type of processing unit represents a simple component separation unit. A single input stream is divided in up to 5 output streams, each one of them has the same temperature and pressure as the input stream, while the distribution of each component in each output stream is specified as fraction of its total input quantity. The phase of each output stream is specified.

The following equations are used:

- Phases of each output stream k:

Spk=Ssetk                                            (3.32)

- Temperatures of each output stream k:

Tpk=Tf1                                               (3.33)

- Pressures of each output stream k:

Ppk=Pf1                                              (3.34)

- Quantities of component i in each output stream k:

Mpki=mkiMf1i                                         (3.35)

- Enthalpies of each output stream k:

Epk=f(Mpki|i=1nc|k=1np,Tpk,Ci|i=1nc)                             (3.36)

3.4.5. Mathematical model for subroutine Unit05

This type of processing unit represents a simple separation unit. A single input stream is divided into 2 output streams, and each one of them has the same temperature and pressure as the input stream, but the presence of each component in one output stream is specified as fraction of the total flow rate of the stream, and the presence of one component in the other output stream, as fraction of the total flow rate of the stream, is specified. Phases of both output streams are specified separately.

The following equations are used:

- Phases of both output streams:

Sp1=Sset1                                            (3.37)

Sp2=Sset2                                            (3.38)

- Temperatures of both output streams:

Tp1=Tf1                                              (3.39)

Tp2=Tf1                                              (3.40)

- Pressures of both output streams:

Pp1=Pf1                                              (3.41)

Pp2=Pf1                                              (3.42)

- Quantities of component i in both output streams:

Mp1i=m1iMf1total[Mf1c/Mf1total-m2c][m1c-m2c]                          (3.43)

where: Mf1total=Σnci=1Mf1i                                                                     (3.43.a)

Mp2i=Mf1i-Mp1i                                      (3.44)

- Enthalpies of both output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                                (3.45)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                                 (3.46)

3.4.6. Mathematical model for subroutine Unit06

This type of processing unit represents a purge or make-up stream, with specified total flow rate of the main stream, or flow rate of one component in the main stream, or one component as fraction of the main stream.

The following equations are used:

- Phases of main stream and purge or make-up stream:

Sp1=Sf1                                              (3.47)

Sp0=Sf1                                              (3.48)

- Temperatures of main stream and purge or make-up stream:

Tp1=Tf1                                              (3.49)

Tp0=Tf1                                              (3.50)

- Pressures of main stream and purge or make-up stream:

Pp1=Pf1                                              (3.51)

Pp0=Pf1                                              (3.52)

- Quantities of component i in main stream and purge or make-up stream:

1. If total flow rate of the main stream is specified:

Mp1i=mMf1i                                           (3.53)

Mp0i=Mf1i-Mp1i                                        (3.54)

2. If flow rate of one component c in the main stream is specified:

Mp1i=Mf1i(except of i=c)                      (3.55)

Mp1c=Mset                                            (3.56)

Mp0i=Mf1i-Mp1i                                        (3.57)

3. If one component c is specified as fraction of the main stream:

Mp1i=Mf1i (except of i=c)                    (3.58)

Mp1c=[Mf1total-Mf1c]m1-m                                   (3.59)

where: Mf1total=Σnci=1Mf1i                                                                    (3.59.a)

Mp0i=Mf1i-Mp1i                                      (3.60)

- Enthalpy of main stream and purge or make-up stream:

1. If total flow rate of the main stream is specified:

Ep1=mEf1                                           (3.61)

Ep0=Ef1-Ep1                                         (3.62)

2. If flow rate of one component c in the main stream is specified or if one component c is specified as fraction of the main stream:

Ep0=f(Mp0c,Tp0,Cc)                                    (3.63)

Ep1=Ef1-Ep0                                         (3.64)

3.4.7. Mathematical model for subroutine Unit07

This type of processing unit represents a heat exchanger, where the temperature of one output stream is specified, while the temperature of the other output stream is calculated. Phases of both input streams remain the same.

The following equations are used:

- Phases of both output streams:

Sp1=SfL                                              (3.65)

where: L=1 or L=2

Sp2=SfL                                              (3.66)

where: L=2 or L=1

- Temperature of 1st output stream:

Tp1=Tset                                             (3.67)

- Pressures of both output streams:

Pp1=PfL                                              (3.68)

where: L=1 or L=2

Pp2=PfL                                              (3.69)

where: L=2 or L=1

- Quantities of component i in both output streams:

Mp1i=MfLi                                             (3.70)

where: L=1 or L=2

Mp2i=MfLi                                             (3.71)

where: L =2 or L=1

- Enthalpies of both output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                                (3.72)

Ep2=Ef1+Ef2-Ep1                                     (3.73)

- Temperature of 2nd output stream is calculated from:

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                                (3.74)

The equation is solved using the method of iterative calculations, until satisfactory convergence is obtained. The input higher stream temperature and the input lower stream temperature are considered to be the upper and lower limits of Tp2, and their mean temperature is the first approximation of Tp2.

If the value of calculated enthalpy using this mean temperature is less than the actual value of Ep2, then it is considered to be the new lower limit of Tp2, while its upper limit remains the same, and their mean value is the new approximation of Tp2.

If the value of calculated enthalpy using this mean temperature is more than the actual value of Ep2, then it is considered to be the new upper limit of Tp2, while its lower limit remains the same, and their mean value is the new approximation of Tp2.

Iterative calculations are continued until satisfactory convergence is obtained between calculated enthalpy and actual value of Ep2, while the last value of Tp2 is the required temperature of output stream for adiabatic operation. If during iterative calculations, the difference between upper and lower value of Tp2 becomes less than 0.1°K, then their mean value is taken as the required output stream temperature Tp2.

3.4.8. Mathematical model for subroutine Unit08

This type of processing unit represents a water cooler, using water as the cooling medium with specified input temperature. Output stream temperature of the process is specified, while its phase remains the same. Cooling water is not present as a process stream, but its output temperature is specified, while its flow rate is calculated.

The following equations are used:

- Phases of both process and non-process output streams:

Sp1=Sf1                                              (3.75)

Sp0=Liq                                             (3.76)

- Temperatures of both process and non-process output streams:

Tp1=Tset                                              (3.77)

Tp0=Tset0                                             (3.78)

- Pressure of process output stream:

Pp1=Pf1                                               (3.79)

- Quantity of component i in the process output stream:

Mp1i=Mf1i                                             (3.80)

- Enthalpies of both process and non-process output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                                (3.81)

Ep0-Ef0 =Ef1-Ep1                                      (3.82)

- Quantity of water in the non-process stream is calculated from:

Ep0-Ef0=f(Mp0w,Tp0,Cw)-g(Mp0w,Tf0,Cw)                 (3.83)

3.4.9. Mathematical model for subroutine Unit09

This type of processing unit represents an evaporator or condenser with specified temperature and pressure of evaporation or condensation, while the temperature of the other output stream, i.e. of the heating or cooling medium, whose the phase remains the same, is calculated. Heat of vaporization or condensation is calculated using Watson equation.

The following equations are used:

- Phases of 1st, 2nd and 3rd output streams:

Sp1=Gas                                            (3.84)

Sp2=Liq                                             (3.85)

Sp3=SfM                                             (3.86)

where: M=1 or M=2

- Temperatures of 1st and 2nd output streams:

Tp1=Tset                                             (3.87)

Tp2=Tset                                            (3.88)

- Pressures of 1st, 2nd and 3rd output streams:

Pp1=Pset                                              (3.89)

Pp2=Pset                                              (3.90)

Pp3=PfM                                              (3.91)

where: M=1 or M=2

- Quantities of component i in the 1st, 2nd and 3rd output streams:

1. For the presence of completely miscible liquids only and between starting and ending of boiling, they are calculated from the set of equations:

Mp1i=MfLi1+Pp1Pvapi[MfLtotalMp1total-1]                                 (3.92)

Mp1total=Σnci=1Mp1i                                         (3.93)

where: Pvapi=f(Tp1)                                                                      (3.92.a)

MfLtotal=Σnci=1MfLi                                                                    (3.92.b)

L=1 or L=2

The equations are solved using the method of iterative calculations, until satisfactory convergence is obtained. MfLtotal and 0 are considered to be the upper and lower limits of Mp1total, and their mean value is the first approximation of Mp1total.

Using the assumed value of Mp1total in eq. (3.92) and (3.93), the quantity of each component Mp1i and the total quantity of gas phase Mp1total are calculated. If the value of calculated Mp1total is less than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new upper limit of Mp1total, while its lower limit remains the same, and their mean value is the new approximation of Mp1total.

If the value of calculated Mp1total is more than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new lower limit of Mp1total, while its upper limit remains the same, and their mean value is the new approximation of Mp1total.

Iterative calculations are continued until satisfactory convergence is obtained between calculated Mp1total and actual value of Mp1total, while the last value of Mp1total is total quantity of gas phase output stream. If during iterative calculations, the difference between upper and lower value of Mp1total becomes less than 0.005, then their mean value is taken as the required total quantity of gas phase output stream Mp1total.

Mp2i=MfLi-Mp1i                                        (3.94)

where: L=1 or L=2

Mp3i=MfMi                                             (3.95)

where: M=1 or M=2

2. For the presence of two immiscible liquids a and b only and between starting and ending of boiling:

Mp1a=MfLa                                            (3.96)

Mp1b=PvapbPvapaMp1a                                        (3.97)

where: L=1 or L=2

Pvapa>Pvapb                                                                      (3.97.a)

Pvapa=f(Tp1)                                                                      (3.97.b)

Pvapb=f(Tp1)                                                                      (3.97.c)

Mp2i=MfLi-Mp1i                                         (3.98)

where: L=1 or L=2

Mp3i=MfMi                                            (3.99)

where: M=1 or M=2

3. For the presence of non-condensable gases only d,e,...... :

Mp1d=MfLd    ,    Mp1e=MfLe    ..........                    (3.100)

Mp2i=MfLi-Mp1i                                     (3.101)

where: L=1 or L=2

Mp3i=MfMi                                          (3.102)

where: M=1 or M=2

4. For combined cases, the above equations are combined accordingly, while for the gas phase it should be always:

Pp1=Σnci=1Ppari                                         (3.103)

- Enthalpies of 1st, 2nd and 3rd output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc,Hi|i=1nc)                         (3.104)

where: Hi=f(Tp1)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                              (3.105)

Ep3=Ef1+Ef2-Ep1-Ep2                               (3.106)

- Temperature of 3rd output stream is calculated from:

Ep3=f(Mp3i|i=1nc,Tp3,Ci|i=1nc)                             (3.107)

The equation is solved using the method of iterative calculations, until satisfactory convergence is obtained. The input maximum stream temperature and the input minimum stream temperature are considered to be the upper and lower limits of Tp3, and their mean temperature is the first approximation of Tp3.

If the value of calculated enthalpy using this mean temperature is less than the actual value of Ep3, then it is considered to be the new lower limit of Tp3, while its upper limit remains the same, and their mean value is the new approximation of Tp3.

If the value of calculated enthalpy using this mean temperature is more than the actual value of Ep3, then it is considered to be the new upper limit of Tp3, while its lower limit remains the same, and their mean value is the new approximation of Tp3.

Iterative calculations are continued until satisfactory convergence is obtained between calculated enthalpy and actual value of Ep3, while the last value of Tp3 is the required temperature of output stream for adiabatic operation. If during iterative calculations, the difference between upper and lower value of Tp3 becomes less than 0.1°K, then their mean value is taken as the required output stream temperature Tp3.

3.4.10. Mathematical model for subroutine Unit10

This type of processing unit represents a heater using saturated steam as the heating medium, or a waste heat steam boiler. The temperature of process stream is specified, while its phase remains the same. Saturated steam, which will be condensed, or water for saturated steam production, are not present as process streams, but the temperature of produced or condensed saturated steam is specified, while its flow rate is calculated. For the case of saturated steam production, the input non-process stream water temperature is specified. Heat of evaporation or condensation of water, is calculated using Watson equation.

The following equations are used:

- Phases of both process and non-process output streams:

1. For the case of steam condensation:

Sp1=Sf1                                            (3.108)

Sp0=Liq                                           (3.109)

2. For the case of steam production:

Sp1=Sf1                                            (3.110)

Sp0=Gas                                          (3.111)

- Temperatures of both process and non-process output streams:

Tp1=Tset                                            (3.112)

Tp0=Tset0                                           (3.113)

- Pressures of both process and non-process output streams:

Pp1=Pf1                                            (3.114)

Pp0=Pvapw                                          (3.115)

where: Pvapw=f(Tp0)                                                                   (3.115.a)

- Quantity for component i in the process stream:

Mp1i=Mf1i                                         (3.116)

- Enthalpies of both process and non-process output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                              (3.117)

Ep0-Ef0 =Ef1-Ep1                                   (3.118)

- Quantity of water (steam) in the non-process stream:

1. For the case of steam condensation:

Ef0-Ep0=f(Mp0w,Hw)                                (3.119)

where: Hw=f(Tp0)                                                                     (3.119.a)

2. For the case of steam production:

Ep0-Ef0=f(Mp0w,Tp0,Cw)-g(Mp0w,Tf0,Cw)                (3.120)

where: Hw=f(Tp0)                                                                     (3.120.a)

3.4.11. Mathematical model for subroutine Unit11

This type of processing unit represents a vaporizer using saturated steam as heating medium with specified temperature. Temperature and pressure of evaporated product is specified. Saturated steam is not present as a process stream, but its flow rate is calculated. Heats of vaporization and condensation of product and steam respectively, are calculated using Watson equation.

The following equations are used:

- Phases of 1st and 2nd process output streams and non-process output stream:

Sp1=Gas                                          (3.121)

Sp2=Liq                                           (3.122)

Sp0=Liq                                           (3.123)

- Temperatures of 1st and 2nd process output streams and non-process output stream:

Tp1=Tset                                           (3.124)

Tp2=Tset                                           (3.125)

Tp0=Tset0                                          (3.126)

- Pressures of 1st and 2nd process output streams and non-process output stream:

Pp1=Pset                                           (3.127)

Pp2=Pset                                           (3.128)

Pp0=Pvapw                                          (3.129)

where: Pvapw=f(Tp0)                                                                  (3.129.a)

- Quantities of component i in the 1st and 2nd process output streams:

1. For the presence of completely miscible liquids only and between starting and ending of boiling, they are calculated from the set of equations:

Mp1i=Mf1i1+Pp1Pvapi[Mf1totalMp1total-1]                               (3.130)

Mp1total=Σnci=1Mp1i                                       (3.131)

where: Pvapi=f(Tp1)                                                                    (3.131.a)

Mf1total=Σnci=1Mf1i                                                                   (3.131.b)

The equations are solved using the method of iterative calculations, until satisfactory convergence is obtained. MfLtotal and 0 are considered to be the upper and lower limits of Mp1total, and their mean value is the first approximation of Mp1total.

Using the assumed value of Mp1total in eq. (3.130) and (3.131), the quantity of each component Mp1i and the total quantity of gas phase Mp1total are calculated. If the value of calculated Mp1total is less than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new upper limit of Mp1total, while its lower limit remains the same, and their mean value is the new approximation of Mp1total.

If the value of calculated Mp1total is more than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new lower limit of Mp1total, while its upper limit remains the same, and their mean value is the new approximation of Mp1total.

Iterative calculations are continued until satisfactory convergence is obtained between calculated Mp1total and actual value of Mp1total, while the last value of Mp1total is total quantity of gas phase output stream. If during iterative calculations, the difference between upper and lower value of Mp1total becomes less than 0.005, then their mean value is taken as the required total quantity of gas phase output stream Mp1total.

Mp2i=Mf1i-Mp1i                                      (3.132)

2. For the presence of two immiscible liquids a and b only and between starting and ending of boiling:

Mp1a=Mf1a                                         (3.133)

Mp1b=PvapbPvapaMp1a                                    (3.134)

where: Pvapa>Pvapb                                                                     (3.134.a)

Pvapa=f(Tp1)                                                                    (3.134.b)

Pvapb=f(Tp1)                                                                    (3.134.c)

Mp2i=Mf1i-Mp1i                                     (3.135)

3. For the presence of non-condensable gases only d,e,...... :

Mp1d=Mf1d    ,    Mp1e=Mf1e    ..........                   (3.136)

Mp2i=Mf1i-Mp1i                                    (3.137)

4. For combined cases, the above equations are combined accordingly, while for the gas phase it should be always:

Pp1=Σnci=1Ppari                                        (3.138)

- Enthalpies of 1st and 2nd process output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc,Hi|i=1nc)                         (3.139)

where: Hi=f(Tp1)                                                                      (3.139.a)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                              (3.140)

Ef0-Ep0 =Ep1+Ep2-Ef1                                (3.141)

- Quantity of steam in the non-process stream is calculated from:

Ef0-Ep0=f(Mp0w,Hw)                                  (3.142)

where: Hw=f(Tp0)                                                                     (3.142.a)

3.4.12. Mathematical model for subroutine Unit12

This type of processing unit represents a condenser, using water as the cooling medium, with specified input temperature. Temperature and pressure of condensed product is specified. Cooling water is not present as a process stream, but its output temperature is specified, while its flow rate is calculated. Heat of condensation of product, is calculated using Watson equation.

The following equations are used:

- Phases of 1st and 2nd process output streams and non-process output stream:

Sp1=Gas                                          (3.143)

Sp2=Liq                                           (3.144)

Sp0=Liq                                           (3.145)

- Temperatures of 1st and 2nd process output streams and non-process output stream:

Tp1=Tset                                           (3.146)

Tp2=Tset                                           (3.147)

Tp0=Tset0                                          (3.148)

- Pressures of 1st and 2nd process output streams:

Pp1=Pset                                           (3.149)

Pp2=Pset                                           (3.150)

- Quantities of component i in the 1st and 2nd process output streams:

1. For the presence of completely miscible liquids only and between starting and ending of boiling, they are calculated from the set of equations:

Mp1i=Mf1i1+Pp1Pvapi[Mf1totalMp1total-1]                               (3.151)

Mp1total=Σnci=1Mp1i                                       (3.152)

where: Pvapi=f(Tp1)                                                                    (3.151.a)

Mf1total=Σnci=1Mf1i                                                                  (3.151.b)

The equations are solved using the method of iterative calculations, until satisfactory convergence is obtained. MfLtotal and 0 are considered to be the upper and lower limits of Mp1total, and their mean value is the first approximation of Mp1total.

Using the assumed value of Mp1total in eq. (3.151) and (3.152), the quantity of each component Mp1i and the total quantity of gas phase Mp1total are calculated. If the value of calculated Mp1total is less than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new upper limit of Mp1total, while its lower limit remains the same, and their mean value is the new approximation of Mp1total.

If the value of calculated Mp1total is more than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new lower limit of Mp1total, while its upper limit remains the same, and their mean value is the new approximation of Mp1total.

Iterative calculations are continued until satisfactory convergence is obtained between calculated Mp1total and actual value of Mp1total, while the last value of Mp1total is total quantity of gas phase output stream. If during iterative calculations, the difference between upper and lower value of Mp1total becomes less than 0.005, then their mean value is taken as the required total quantity of gas phase output stream Mp1total.

Mp2i=Mf1i-Mp1i                                      (3.153)

2. For the presence of two immiscible liquids a and b only and between starting and ending of boiling:

Mp1a=Mf1a                                          (3.154)

Mp1b=PvapbPvapaMp1a                                      (3.155)

where: Pvapa>Pvapb                                                                     (3.155.a)

Pvapa=f(Tp1)                                                                    (3.155.b)

Pvapb=f(Tp1)                                                                    (3.155.c)

Mp2i=Mf1i-Mp1i                                      (3.156)

3. For the presence of non-condensable gases only d,e,...... :

Mp1d=Mf1d    ,    Mp1e=Mf1e    ..........                     (3.157)

Mp2i=Mf1i-Mp1i                                      (3.158)

4. For combined cases, the above equations are combined accordingly, while for the gas phase it should be always:

Pp1=Σnci=1Ppari                                          (3.159)

- Enthalpies of 1st and 2nd process output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc,Hi|i=1nc)                         (3.160)

where: Hi=f(Tp1)                                                                      (3.160.a)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                              (3.161)

Ep0-Ef0 =Ef1-Ep1-Ep2                                (3.162)

- Quantity of steam in the non-process stream is calculated from:

Ep0-Ef0=f(Mp0w,Tp0,Cw)-g(Mp0w,Tf0,Cw)                (3.163)

3.4.13. Mathematical model for subroutine Unit13

This type of processing unit represents a single stage evaporator or one stage of a multistage evaporator. Temperature of water evaporation is specified, while the quantity of evaporated water is calculated and considered as the second product output stream. Heats of evaporation and condensation of water, are calculated using Watson equation.

The following equations are used:

- Phases of 1st, 2nd and 3rd output streams:

Sp1=SfL                                           (3.164)

where: L=1 or L=2

Sp2=Gas                                           (3.165)

Sp3=Liq                                           (3.166)

- Temperatures of 1st, 2nd and 3rd output streams:

Tp1=Tset                                           (3.167)

Tp2=Tset                                           (3.168)

Tp3=Tset3                                          (3.169)

- Pressures of 1st, 2nd and 3rd output streams:

Pp1=Pvapw                                          (3.170)

where: Pvapw=f(Tp1)                                                                   (3.170.a)

Pp2=Pvapw                                          (3.171)

where: Pvapw=f(Tp2)                                                                  (3.171.a)

Pp3=Pvapw3                                         (3.172)

where: Pvapw3=f(Tp3)                                                                 (3.172.a)

- Quantities of component i in the 1st (except of water) and in the 3rd output stream:

Mp1i=MfLi (except of i=w)                  (3.173)

where: L=1 or L=2

Mp3w=MfKw                                         (3.174)

where: K=1 or K=2

- Enthalpy of 3rd output stream:

Ep3=f(Mp3w,Tp3,Cw)                                  (3.175)

- Quantities of water in the 2nd and the 1st output streams:

Mp2w=[EfL-E]+[EfK-Ep3]Hw                             (3.176)

where: E=f(MfLi|i=1nc,Tp1,Ci|i=1nc)                                                    (3.176.a)

Hw=f(Tp1)                                                                      (3.176.b)

K=1 or K=2 and L=1 or L=2

Mp1w=MfLw-Mp2w                                    (3.177)

where: L=1 or L=2

- Enthalpies of 1st and 2nd output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                              (3.178)

Ep2=f(Mp2w,Tp2,Cw,Hw)                               (3.179)

3.4.14. Mathematical model for subroutine Unit14

This type of processing unit represents a single stage evaporator or one stage of a multi stage evaporator. Temperature of water evaporation is specified. The required quantity of one component in the main process output stream is specified as fraction of its total stream flow rate. Saturated steam required as heating medium, is not present as process stream, but its quantity is calculated. Heats of evaporation and condensation of water are calculated using Watson equation.

The following equations are used:

- Phases of 1st and 2nd process output streams and non-process output stream:

Sp1=Liq                                           (3.180)

Sp2=Gas                                          (3.181)

Sp0=Liq                                           (3.182)

- Temperatures of 1st and 2nd process output streams and non-process output stream:

Tp1=Tset                                           (3.183)

Tp2=Tset                                           (3.184)

Tp0=Tset0                                          (3.185)

- Pressures of 1st and 2nd process output streams and non-process output stream:

Pp1=Pvapw                                          (3.186)

where: Pvapw=f(Tp1)                                                                   (3.186.a)

Pp2=Pvapw                                          (3.187)

where: Pvapw=f(Tp2)                                                                  (3.187.a)

Pp0=Pvapw0                                         (3.188)

where: Pvapw0=f(Tp0)                                                                 (3.188.a)

- Quantities of component i in the 1st and 2nd process output streams:

Mp1i=Mf1i (except of i=w)                   (3.189)

Mp2w=Mf1total−Mf1cm                                     (3.190)

Mp1w=Mf1w-Mp2w                                      (3.191)

- Enthalpies of 1st and 2nd process output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                              (3.192)

Ep2=f(Mp2w,Tp2,Cw,Hw)                               (3.193)

where: Hw=f(Tp2)                                                                     (3.193.a)

Ep0-Ef0 =Ef1-Ep1+Ep2                                (3.194)

- Quantity of steam in the non-process stream is calculated from:

Ep0-Ef0=f(Mp0w,Tp0,Cw)-g(Mp0w,Tf0,Cw)+h(Mp0w,Hw)      (3.195)

where: Hw=f(Tf0)                                                                      (3.195.a)

3.4.15. Mathematical model for subroutine Unit15

This type of processing unit represents a drier, operating with hot air, with specified input temperatures of product and air, while the quantity of evaporated water, carried by the cold air stream, is calculated, assuming adiabatic operation at 1atm abs. Heat of evaporation of water, is calculated using Watson equation.

The following equations are used:

- Phases of both output streams:

Sp1=SfL                                           (3.196)

where: L=1 or L=2

Sp2=SfL                                          (3.197)

where: L=1 or L=2

- Temperatures of both output streams:

Tp1=Tset1                                           (3.198)

Tp2=Tset2                                          (3.199)

- Pressures of both output streams:

Pp1=PfL                                           (3.200)

where: L=1 or L=2

Pp2=PfL                                          (3.201)

where: L=1 or L=2

- Quantities of component i in both output streams:

Mp1i=MfLi (except of i=w)                (3.202)

where: L=1 or L=2

Mp2i=MfLi(except of i=w)                 (3.203)

where: L=1 or L=2

Mp2w=MfLw+Mwev                                   (3.204)

where: L=1 or L=2 and

from equation:

[Ef1+Ef2]−[E1+E2]=f(Mwev,Tp2,Cw)-g(Mwev,Tp1,Cw)+h(Mwev,Hw) (3.205)

where: E1=f(MfLi|i=1nc,Tp1,Ci|i=1nc)                                                  (3.205.a)

and L=1 or L=2

E2=f(MfLi|i=1nc,Tp2,Ci|i=1nc)                                                  (3.205.b)

and L=1 or L=2

Hw=f(Tp1)                                                                      (3.205.c)

Mp1w=MfLw-Mp2w                                    (3.206)

where: L=1 or L=2

- Enthalpies of both output streams:

Ep1=f(Mf1i|i=1nc,Tp1,Ci|i=1nc)                               (3.207)

Ep2=Ef1+Ef2-Ep1                                   (3.208)

3.4.16. Mathematical model for subroutine Unit16

This type of processing unit represents a drier, operating with hot air, with specified final product humidity. Hot air is not present as process stream, but its input and output temperatures are specified, while its quantity is calculated, assuming adiabatic operation at 1atm abs. Heat of evaporation of water, is calculated using Watson equation.

The following equations are used:

- Phases of both process and non-process output streams:

Sp1=Sf1                                            (3.209)

Sp0=Gas                                           (3.210)

- Temperatures of both process and non-process output streams:

Tp1=Tset                                           (3.211)

Tp0=Tset0                                          (3.212)

- Pressure of process output stream:

Pp1=Pf1                                            (3.213)

- Quantity of component i in the process output stream:

Mp1i=Mf1i (except of i=w)                 (3.214)

Mp1w=m1Mf1total-w1-m1                                     (3.215)

where: Mf1total-w=Σnci=1Mf1i-Mf1w                                                        (3.215.a)

- Enthalpy of process output stream:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                               (3.216)

- Quantity of air in the non-process output stream:

Mp0w=m0Mp0total+Mf1w-Mp1w                            (3.217)

where: Mp0total=[Ef1-Ep1-Ewev][m0(Ew'+Hwp-Hwf)+(1-m0)(0.212Eo'+0.788En')]  (3.217.a)

Ewev=f(Mf1w-Mp1w,Tp1Cwl)+g(Mf1w-Mp1w,Hw)+

+h(Mf1w-Mp1w,Tp0,Cwg)-e(Mf1w-Mp1w,Tp1,Cwg)  (3.217.b)

(3.217.c)

(3.217.d)

(3.217.e)

Ew'=f(Tp0,Cw)-g(Tf0,Cw)                                               (3.217.f)

Eo'=f(Tp0,Co)-g(Tf0,Co)                                                (3.217.g)

En'=f(Tp0,Cn)-g(Tf0,Cn)                                                 (3.217.h)

Mp0o=0.212[1-m0]Mp0total                               (3.218)

Mp0n=0.788[1-m0]Mp0total                               (3.219)

3.4.17. Mathematical model for subroutine Unit17

This type of processing unit represents a water cooling system with specified temperature and relative humidity of hot air output stream, while the quantity of evaporated water carried by the hot air output stream and the cold water output stream temperature are calculated assuming adiabatic operation at 1 atm abs. Heat of evaporation of water, is calculated using Watson equation.

The following equations are used:

- Phases of both output streams:

Sp1=SfL                                            (3.220)

where: L=1 or L=2

Sp2=SfL                                           (3.221)

where: L=1 ή L=2

- Temperature of 2nd output stream:

Tp2=Tset2                                           (3.222)

- Pressures of both output streams:

Pp1=PfL                                            (3.223)

where: L=1 ή L=2

Pp2=PfL                                            (3.224)

where: L=1 or L=2

- Quantities of component i in both output streams:

Mp2i=MfLi(except of i=w)                   (3.225)

where: L=1 or L=2

Mp2w=mPvapwPtotal-Pvapw(Mp2o+Mp2n)                       (3.226)

where: Pvapw=f(Tp2)                                                                  (3.226.a)

Ptotal=101.33 KNm2                                                     (3.226.b)

Mp1w=Mf1w+Mf2w-Mp2w                                (3.227)

- Enthalpies of both output streams:

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc,Hw)                            (3.228)

where: Hw=f(Tp2)                                                                     (3.228.a)

Ep1=Ef1+Ef2-Ep2                                  (3.229)

- Temperature of 1st output stream is calculated from:

Ep1=f(Mp1w,Tp1,Cw)                                  (3.230)

3.4.18. Mathematical model for subroutine Unit18

This type of processing unit represents a water cooling system with specified final temperature of cold water. Air is not present as process stream, but temperatures and relative humidities of cold air input stream and hot air output stream are specified, while its quantity is calculated assuming adiabatic operation at 1 atm abs. Heat of evaporation of water, is calculated using Watson equation.

The following equations are used:

- Phases of both process and non-process output streams:

Sp1=Sf1                                            (3.231)

Sp0=Gas                                           (3.232)

- Temperatures of both process and non-process output streams:

Tp1=Tset                                           (3.233)

Tp0=Tset0                                          (3.234)

- Pressure of process output stream:

Pp1=Pf1                                           (3.235)

- Quantity of component i in the process output stream and quantity of air in the non-process output stream:

Mp1i=Mf1i (except of i=w)                 (3.236)

Mp0w=m0pPvapwpPtotal-Pvapwp(Mp0o+Mp0n)                      (3.238)

where: Mp0o+Mp0n=                                                                        (3.238.a)

=Ef1-E(0.212Eo'+0.788En'+m0pPvapwpPtotal-Pvapwp[Ewp'+Hwp-Ewev']-m0fPvapwfPtotal-Pvapwf[Ewf'+Hwf-Ewev'])

E=f(Mf1w,Tp1,Cw)                                                           (3.238.b)

Eo'=f(Tp0,Co)-g(Tf0,Co)                                                (3.238.c)

En'=f(Tp0,Cn)-g(Tf0,Cn)                                                (3.238.d)

Pvapwf=f(Tf0)                                                                  (3.238.e)

Pvapwp=f(Tp0)                                                                  (3.238.f)

Ptotal=101.33 KNm2                                                     (3.238.g)

Hwf=f(Tf0)                                                                     (3.238.h)

Hwp=f(Tp0)                                                                     (3.238.i)

Ewev'=f(Tp1,Cw)                                                              (3.238.j)

Ewp'=f(Tp0,Cw)                                                              (3.238.k)

Ewf'=f(Tf0,Cw)                                                                (3.238.l)

Mp1w=Mf1w+m0fPvapwfPtotal-Pvapwf(Mp0o+Mp0n)-Mp0w           (3.239)

where: Pvapwf=f(Tf0)                                                                  (3.239.a)

Ptotal=101.33 KNm2                                                     (3.239.b)

Mp0o=0.212(Mp0o+Mp0n)                               (3.240)

Mp0n=0.788(Mp0o+Mp0n)                               (3.241)

- Enthalpy of process output stream:

Ep1=f(Mp1w,Tp1,Cw)                                 (3.242)

3.4.19. Mathematical model for subroutine Unit19

This type of processing unit represents a simple light and heavy phase separation liquid/liquid system, operating at constant temperature and constant pressure. Separation factors are considered to be constants.

The following equations are used:

- Phases of both output streams:

Sp1=Liq                                           (3.243)

Sp2=Liq                                           (3.244)

- Temperatures of both output streams:

Tp1=Tf1                                            (3.245)

Tp2=Tf1                                            (3.246)

- Pressures of both output streams:

Pp1=Pf1                                            (3.247)

Pp2=Pf1                                            (3.248)

- Quantities of component i in both output streams:

1. For each component consisting the heavy phase:

Mp1i=m2[Mf1totallight-m1Mf1total][1-m1-m2]Mf1totalheavyMf1i                (3.249)

Mp2i=Mf1i-Mp1i                                      (3.250)

2. For each component consisting the light phase:

Mp2i=m1[Mf1totalheavy-m2Mf1total][1-m1-m2]Mf1totallightMf1i              (3.251)

Mp1i=Mf1i-Mp2i                                   (3.252)

where: Mf1total=Σnci=1Mf1i                                                                     (3.253)

Mf1totallight  = total quantity of light phase in the input stream

Mf1totalheavy = total quantity of heavy phase in the input stream

- Enthalpies of both output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                             (3.254)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                              (3.255)

3.4.20. Mathematical model for subroutine Unit20

This type of processing unit represents an absorber or stripper or liquid/liquid extractor or solid/liquid extractor, operating at constant temperature and constant pressure. The recovery of each component existing in one output stream, as fraction of its total input in the proper input stream, is specified.

The following equations are used:

- Phases of both output streams:

Sp1=SfL                                           (3.256)

where: L=1 or L=2

Sp2=SfL                                           (3.257)

where: L=1 or L=2

- Temperatures of both output streams:

Tp1=Tset                                           (3.258)

Tp2=Tset                                           (3.259)

- Pressures of both output streams:

Pp1=Pset                                           (3.260)

Pp2=Pset                                           (3.261)

- Quantities of component i in both output streams:

MpNi=miMfLi+MfKi                                      (3.262)

where: N=1 or N=2

L=1 or L=2

K=1or K=2

MpMi=Mf1i+Mf2i-MpNi                                  (3.263)

where: M=1 or M=2

N=1 ή N=2

- Enthalpies of both output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                             (3.264)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                              (3.265)

3.4.21. Mathematical model for subroutine Unit21

This type of processing unit represents a vapour/liquid separation system with specified pressure,	and it calculates the output streams temperature assuming adiabatic operation, or it sets the output streams temperature assuming isothermal operation, or it sets the output streams temperature equal to a specified value.

The following equations are used:

- Phases of both output streams:

Sp1=Gas                                          (3.266)

Sp2=Liq                                           (3.267)

- Temperatures of both output streams:

1. In the case of isothermal operation:

Tp1=Tf1                                            (3.268)

Tp2=Tf1                                            (3.269)

2. In the case where output streams temperature is specified:

Tp1=Tset                                           (3.270)

Tp2=Tset                                          (3.271)

3. In the case of adiabatic operation, by solving a set of equations given below, for calculation of quantities of components i in both output streams, in the case of adiabatic operation.

- Phases of both output streams:

Pp1=Pset                                           (3.272)

Pp2=Pset                                           (3.273)

- Quantities of component i in both output streams in the case of isothermal operation or in the case where output streams temperature is specified:

1. For the presence of completely miscible liquids only and between starting and ending of boiling, they are calculated from the set of equations:

Mp1i=Mf1i1+Pp1Pvapi[Mf1totalMp1total-1]                               (3.274)

Mp1total=Σnci=1Mp1i                                       (3.275)

where: Pvapi=f(Tp1)                                                                    (3.274.a)

Mf1total=Σnci=1Mf1i                                                                   (3.274.b)

The equations are solved using the method of iterative calculations, until satisfactory convergence is obtained. MfLtotal and 0 are considered to be the upper and lower limits of Mp1total, and their mean value is the first approximation of Mp1total.

Using the assumed value of Mp1total in eq. (3.274) and (3.275), the quantity of each component Mp1i and the total quantity of gas phase Mp1total are calculated. If the value of calculated Mp1total is less than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new upper limit of Mp1total, while its lower limit remains the same, and their mean value is the new approximation of Mp1total.

If the value of calculated Mp1total is more than the assumed value of Mp1total, then the assumed value of Mp1total is considered to be the new lower limit of Mp1total, while its upper limit remains the same, and their mean value is the new approximation of Mp1total.

Iterative calculations are continued until satisfactory convergence is obtained between calculated Mp1total and actual value of Mp1total, while the last value of Mp1total is total quantity of gas phase output stream. If during iterative calculations, the difference between upper and lower value of Mp1total becomes less than 0.005, then their mean value is taken as the required total quantity of gas phase output stream Mp1total.

Mp2i=Mf1i-Mp1i                                     (3.276)

2. For the presence of two immiscible liquids a and b only and between starting and ending of boiling:

Mp1a=Mf1a                                         (3.277)

Mp1b=PvapbPvapaMp1a                                     (3.278)

where: Pvapa>Pvapb                                                                    (3.278.a)

Pvapa=f(Tp1)                                                                    (3.278.b)

Pvapb=f(Tp1)                                                                    (3.278.c)

Mp2i=Mf1i-Mp1i                                      (3.279)

3. For the presence of non-condensable gases only d,e,...... :

Mp1d=Mf1d    ,    Mp1e=Mf1e    ..........                     (3.280)

Mp2i=Mf1i-Mp1i                                      (3.281)

4. For combined cases, the above equations are combined accordingly, while for the gas phase it should be always:

Pp1=Σnci=1Ppari                                         (3.282)

- Quantities of component i in both output streams in the case of adiabatic operation:

The previous equations (3.274) to (3.282), are solved using assumed value for  which must be verified by equation:

Ef1=Ep1+Ep2                                    (3.283)

where: Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc,Hi|i=1nc)                                        (3.283.a)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                                                (3.283.b)

Hi=f(Tp1)                                                                      (3.283.c)

Tp2=Tp1                                                                         (3.283.d)

The equation is solved using the method of iterative calculations, until satisfactory convergence is obtained. To determine an upper and a lower limit of Tp1, it is assumed at the beginning that the value of Tp1 is that of the input stream i.e.Tf1.

If the value of calculated enthalpy using this temperature is less than the actual value of Ef1, then the assumed value is considered to be the new lower limit of Tp1, while its upper limit is assumed to be 1.2(Tp1), and their mean value is the new approximation of Tp1.

If the value of calculated enthalpy using this temperature is more than the actual value of Ef1, then the assumed value is considered to be the new upper limit of Tp1, while its lower limit is assumed to be 0.8(Tp1), and their mean value is the new approximation of Tp1.

Calculations are continued with the same way until it is confirmed that the required temperature Tp1 lies between the estimated previously upper and lower limit, and their mean temperature is the actual first approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is less than the actual value of Ef1, then it is considered to be the new lower limit of Tp1, while its upper limit remains the same, and their mean value is the new approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is more than the actual value of Ef1, then it is considered to be the new upper limit of Tp1, while its lower limit remains the same, and their mean value is the new approximation of Tp1.

Iterative calculations are continued until satisfactory convergence is obtained between calculated enthalpy and actual value of Ef1, while the last value of Tp1 is the required temperature of output stream for adiabatic operation. If during iterative calculations, the difference between upper and lower value of Tp1 becomes less than 0.1°K, then their mean value is taken as the required output stream temperature Tp1.

- Enthalpies of both output streams:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc,Hi|i=1nc)                        (3.284)

where: Hi=f(Tp1)                                                                      (3.284.a)

Ep2=f(Mp2i|i=1nc,Tp2,Ci|i=1nc)                             (3.285)

3.4.22. Mathematical model for subroutine Unit22

This type of processing unit represents a simple reactor for steady transformation and if the temperature of the input stream is specified, it calculates the output stream temperature assuming adiabatic operation, or it sets the output stream temperature assuming isothermal operation, or it sets the output stream temperature equal to a specified value. Output stream pressure is equal to the input stream pressure. Reaction is considered to take place at 1 atm abs. Heat of reaction is calculated from heats of generation of the reaction components, while the conversion factor of a key component is specified.

The following equations are used:

- Phase of output stream:

Sp1=Sset                                           (3.286)

- Temperature of output stream:

1. In the case of isothermal operation:

Tp1=Tf1                                             (3.287)

2. In the case where output stream temperature is specified:

Tp1=Tset                                           (3.288)

- Pressure of output stream:

Pp1=Pf1                                            (3.289)

- Quantity of component i in the output stream:

Mp1i=Mf1i+mMp1cFi                                   (3.290)

where: Fi = stoichiometric factor of component i

- Enthalpy of output stream:

1. In the case of isothermal operation or in the case where output stream temperature is specified:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                              (3.291)

2. In the case of adiabatic operation:

Ep1=Ef1+Hr                                        (3.292)

- Temperature of output stream in the case of adiabatic operation is calculated from:

Ep1=f(Mp1i|i=1nc,Tp1,Ci|i=1nc)                            (3.293)

The equation is solved using the method of iterative calculations, until satisfactory convergence is obtained. To determine an upper and a lower limit of Tp1, it is assumed at the beginning that the value of Tp1 is 1.1(Tf1) for exothermic reaction or 0.9(Tf1) for endothermic reaction.

If the value of calculated enthalpy using this temperature is less than the actual value of Ep1, then the assumed value is considered to be the new lower limit of Tp1, while its upper limit is assumed to be 1.2(Tp1), and their mean value is the new approximation of Tp1.

If the value of calculated enthalpy using this temperature is more than the actual value of Ep1, then the assumed value is considered to be the new upper limit of Tp1, while its lower limit is assumed to be 0.8(Tp1), and their mean value is the new approximation of Tp1.

Calculations are continued with the same way until it is confirmed that the required temperature Tp1 lies between the estimated previously upper and lower limit, and their mean temperature is the actual first approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is less than the actual value of Ep1, then it is considered to be the new lower limit of Tp1, while its upper limit remains the same, and their mean value is the new approximation of Tp1.

If the value of calculated enthalpy using this mean temperature is more than the actual value of Ef1, then it is considered to be the new upper limit of Tp1, while its lower limit remains the same, and their mean value is the new approximation of Tp1.

Iterative calculations are continued until satisfactory convergence is obtained between calculated enthalpy and actual value of Ep1, while the last value of Tp1 is the required temperature of output stream for adiabatic operation. If during iterative calculations, the difference between upper and lower value of Tp1 becomes less than 0.1°K, then their mean value is taken as the required output stream temperature Tp1.

3.5. VAPOUR PRESSURE CALCULATIONS

Vapour pressure at temperature T is calculated using Antoine equation, which provides accurate results especially in processing agricultural products.

ln Pvapi = Ai - BiT + Ci                                      (3.294)

where: Αi , Bi , Ci  = constants of Antoine equation for component i

T = actual stream temperature

3.6. CALCULATION OF HEAT OF VAPORIZATION

Heat of evaporation at temperature T is calculated using Watson equation, which provides accurate results especially in processing agricultural products.

Hi = Hbi[Tci - TTci - Tbi]0.38                                   (3.295)

where: Hbi = heat of evaporation of component i at normal S.T.P. conditions

Tbi = normal evaporation temperature of component i

Tci = critical temperature of component i

T = actual stream temperature