4. PRODUCTS OF AGRICULTURAL INDUSTRIES

# METHODS OF CALCULATIONS

For the case of agricultural industries, where there are not enough information about physical properties of agricultural products, it is assumed that most agricultural products may be considered as soluble solids or insoluble solids in water, or in some cases as liquids, where no phase change takes place during processing. On the basis of this assumption, a second data bank was developed to include many agricultural products. The main interesting physical properties existing in this data bank, are the constants of specific heat capacity equations in solid or liquid phase.

The usefulness of this model of agricultural products could be understood if we take in consideration that during processing agricultural products, there are only changes in compositions of soluble or insoluble solids or even independent liquids in water. For example, tomato juice is considered to be a solution of tomato soluble solids in water, coffee is considered to be a mixture of coffee soluble solids in water and coffee insoluble solids, while olives products are liquid olive oil and olive solids. In all cases, soluble solids, insoluble solids or independent liquids, are assumed to keep their specific heat during processing, while specific heat capacities calculation are carried out on the basis of weighted calculations of mixture, since the only change, which takes place during processing, is the change in composition.

Agricultural products, existing in the agricultural products data bank, are given in Table (4.1). To evaluate the specific heat capacity equation constants for each one of them, the mentioned agricultural products are considered to have the composition shown in this Table (4.1). As a first approximation, the specific heat capacity of each one was calculated taking into consideration their composition. Then, the calculated value was verified experimentally, using solutions or suspensions of different compositions and measuring their actual heat capacity. Experimental values where found to be close enough to the calculated ones.

To evaluate specific heat capacity equation constants, data were taken from Coulson and Richardson [28], Perry and Chilton [29] and other references related with food industry [30, 31, 32, 33].

## Table (4.1).  Agricultural products existing in the physical properties data bank and their composition (mean values %).

__________________________________________________________________________

| ### PRODUCT NAME | Sugars (1) | Organic acids (2) | Ammino acids (3) | Minerals (4) | Sodium chloride | Phenolics | Caffeine | Cellulose & fibres | Insoluble proteins | Pectins | Vegetable oils |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cherry insoluble solids (assumed) |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Cherry soluble solids | 82 | 8 | 6 | 4 |  |  |  |  |  |  |  |
| Coffee insoluble solids | 15 |  |  | 2 |  |  |  | 38 | 20 |  | 25 |
| Coffee soluble solids | 64 | 14 | 5 | 8 |  | 5 | 4 |  |  |  |  |
| Grape insoluble solids (assumed) |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Grape soluble solids | 88 | 6 | 4 | 2 |  |  |  |  |  |  |  |
| Lemon insoluble solids |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Lemon soluble solids | 23 | 62 | 10 | 5 |  |  |  |  |  |  |  |
| Olive oil |  |  |  |  |  |  |  |  |  |  | 100 |
| Olive solids | 68 |  | 6 | 5 |  |  |  | 21 |  |  |  |
| Orange insoluble solids (assumed) |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Orange soluble solids | 67 | 19 | 10 | 4 |  |  |  |  |  |  |  |
| Pear insoluble solids (assumed) |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Pear soluble solids | 75 | 9 | 12 | 4 |  |  |  |  |  |  |  |
| Tomato insoluble solids (assumed) |  |  |  |  |  |  |  | 80 | 10 | 10 |  |
| Tomato soluble solids | 60 | 8 | 20 | 10 | 2 |  |  |  |  |  |  |

(1). Mainly Glucose, Fructoze, Sucroze etc.

(2). Mainly Citric acid, Malic acid etc.

(3). Mainly Glutamic acid and soluble proteins

(4). Mainly Sodium, Potasium, Calsium, etc.