1. INTRODUCTION

## LITERATURE SURVEY

By process simulation we usually mean the structure of a model, which represents the behaviour of a process. This model may be the result of analytical and mathematical representation of a process or its empirical representation in the form of empirical equations. The main criterion for the model selection is the examination of its behaviour in comparison with the behaviour of the real process, while the assessment of this selection depends on the “coincidence” of the real situations with the model situations.

Process simulation has been widely used and from many years by chemical engineers to study different processes existing in industries of chemical engineering interest. Term “simulation” has a special meaning when it is combined with computers, since computer aided simulation and design enables the designer engineer to consider different processes and more alternative processing schemes, in a very short time interval, if compared with manual calculations.

Processes simulation is widely used either for their preliminary evaluation or for the analysis of existing processing plants. Simulation systems used for processes preliminary evaluation must provide a large number of information from a small number of input data. Also these simulation systems must provide the user with reliable results, which can be used as data for processing plants design.

Flow-sheet is a very important type of simulation for preliminary process evaluation. Flow-sheet itself is the key-document in evaluation, design and analysis of processes. It shows the arrangement of processing units selected to carry out the process, the stream connections, stream flow rates and compositions and the operating conditions. It is actually the diagrammatic model of the process, which includes:

- A simple schematic diagram of the process showing all unit operations and chemical reactions.

- Streams flow rates and compositions, as a result of overall mass balance and components mass balance applied over the complete process and each individual processing unit.

- A list of all available data such as temperatures, pressures and other necessary physical data.

- Streams enthalpies as optional information, as a result of energy balance over the complete process and/or each individual processing unit.

Flow-sheeting calculations techniques require material balance calculations combined with other equations that arise from process and equipment constraints such as:

- Process stoichiometry, and reactor conversions

- Chemical equilibria

- Physical equilibria

- Energy balance constraints

Manual flow-sheeting calculations can be tedious and time consuming, when the process is large or complex. Computer aided flow-sheeting calculations programs are being increasingly used. Their use enables the designer engineer to consider different processes and more alternative processing schemes during his search for the best process and optimum process conditions.

Some industries, universities and computer software companies have produced a large number of computer programs performing flow-sheeting calculations. Some of them provide solutions for a preliminary process evaluation, while some others provide processing units design too.

A processing system is usually defined be a set o equations accompanied by a set of constraints. Equations have usually algebraic form for steady state operations and it is required to find out the values of variables xi so that:

fi ( xi ) = 0                                                 (1.1)

where: i = 1,2,3,.......n   is the number of variables

j = 1,2,3,.......m   is the number of equations

A system of processing units defined by eq. (1.1), usually includes a higher number of variables n than the number of equations m (i.e. n>m) and it is necessary that a number of (n – m) variables to be defined either by the user or by a certain methodology, before the evaluation of the rest m variables from the m equations.

Two main methods exist to solve the problem defined by eq. (1.1). The first and most common method is the use of separated calculation subroutines (modules), while the second method is the development of a procedure to solve the system of algebraic equation.

1.1. METHOD OF SEPARATED CALCULATION SUBROUTINES

By this method, all equations defining each processing unit are included in a different calculation subroutine (module). For processing unit number r there is a number of nr variables, which is higher than the number of equations mr. The rest (nr – mr) variables are decision variables and they are entered by the user as input data for the subroutine, which will calculate the rest mr state variables.

Fig. (1.1). Typical organization of a computer program for flow-sheeting calculations, using the method of different calculation subroutines.

These programs have usually the form and construction shown in Fig. (1.1) and they consist of:

- The main executive program, which controls the order of calculations and the flow of information from and to the subroutines.

- A library of subroutines, which simulate the different processing units and calculate for each unit the output streams data (state variables) from the input streams data and from a number of other data introduced by the user (decision variables).

- Convergence subroutines, which locate the recycle streams and calculate the recycle flows data, checking their convergence.

- Physical properties data banks.

- Special type subroutines for special calculations, such as thermodynamic calculations, transport calculations etc.

There are some computer simulation systems, which carry out optimization and design calculations for the processing units. Such systems include special routines for design calculations and data banks for cost estimations. Obviously such systems provide a very accurate system simulation.

A summary of some important flow-sheeting calculations programs is given in Table (1.1). Crowe et al. (1971) [1], Westerburg et al. (1979) [2] and Benedek (1980) [3] give detailed information about these programs.

1.1.1. SYSTEM PRESENTATION

Each process system is represented for use by computer programs performing flow-sheeting calculations by transformation of the process flow diagram, which shows the order of different processing units and stream connections, into information block flow diagram. Every block represents a different calculation subroutine, i.e. represents a certain processing unit.

Table (1.1). Summary of some important computer programs performing flow-sheeting calculations.

| NAME | DEVELOPER | REFERENCE |
| --- | --- | --- |
| GEMCS | McMasters University | -Johnson and Associates |
|  | U.K. | "GEMCS" manual (1970) |
| FLOWPACK | Imperial Chemical | -"FLOWPACK" users manual |
|  | Industries Ltd | (1970) |
|  | U.K. | -Davies (1971) [4] |
| FLOWTRAN | Monsato Co and CACHE | -"An introduction to |
|  | U.S.A. | FLOWTRAN", Monsato Co |
|  |  | (1970) |
|  |  | -Seader et al.(1977) [5] |
|  |  | -Clark (1977) [6] |
| PROCESS DESIGN SYSTEM | General Electric Co | -"Process Design System" |
|  | U.S.A. | users guide, |
|  |  | General Electric Co (1970) |
| CHIPS | IBM Corporation | -"CHIPS" users manual |
|  | U.S.A. |  |
| CONCEPT | CAD Centre, Cambridge | -"CONCEPT" users manual |
|  | U.K. | -Hutchinson and Leesley |
|  |  | (1973) [7] |
| PROCESS | Simulation Science Inc. | -"PROCESS" users and |
|  |  | Application manual |
| DESIGN II | CHEMSHARE Co | -"DESIGN II" users manual |
|  | U.S.A. |  |
| ASPEN PLUS | Aspen Technology Inc. | -"ASPEN PLUS" users |
|  | Cambridge, | manual |
|  | Massachusetts, U.S.A. |  |

Processing units in which there is no change in composition, temperature or pressure of products, are ignored. On the other hand, some processes, which are not presented in the process flow diagram, but causes changes in composition, temperature or pressure of products, must be presented in the form of blocks in the information block flow diagram.

Calculation subroutines representing these blocks can vary in calculation complexity from traditional unit operation calculations to calculations based on strictly empirical equations, which simulate several unit operations {Raatz and Spellman (1962) [8]}. Connection of blocks with lines and arrows shows the flow of information from and to calculation subroutines.

An example of such transformation of basic flow diagram into information block flow diagram is shown in Fig. (1.2).

1. Mixer

2. Steam Heater

3. Reactor

4. Water Condenser

5. Scrubber

6. Mixer

7. Acetone Column

8. Isopropanol Column

Fig. (1.2). Transformation of process flow diagram into block flow diagram, for the production of acetone by dehydrogenation of isopropanol.

(A) Process flow diagram.

(B) Information block flow diagram.

1.1.2. PROCESSES TOPOLOGY DEFINITION

Process topology is usually defined by an array S, which includes the names or numbers of streams in the diagram, in combination with two other arrays for each processing unit, one for the input streams F, which includes the identification of each input stream to the unit and one for the output streams P, which includes the identification of each output stream from the unit. By the word “identification” we usually mean the name and/or the number of each stream, defining so the processes topology.

Processes topology is possible to be defined by a table with elements the numbers zero and one, so that each block occupies a raw and a column. Elements with a value one indicate the existence of stream information from raw block to the corresponding column block of the information block flow diagram. This method is shown in Fig. (1.3).

Both methods for topology definition are equivalent, since both offer the same information. By the method of table representation, there is no need of any topology array S, since each stream is defined directly from its starting point and its destination. On the other hand, the existence of many blocks in the information block flow diagram increases the table size.

|  |  |  | D E S T I N A T I O N |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # Feed | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | # Prod |
|  | # Feed | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| S | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| T | 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| A | 3 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| R | 4 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| T | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| I | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| N | 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| G | 8 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
|  | # Prod | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Fig. (1.3). Definition of process topology, using the method of table with elements 0 and 1.

1.1.3. REQUIRED INPUT DATA

Except o topology definition it is necessary for some other data to be defined.

- A data array U, which includes all necessary data for each processing unit, including all data defined by the user (decision variables).

- A data array D, which includes all required data for all input streams to the system.

- Some other information, which must be defined by the user, such as information about the way of calculation of physical properties etc.

1.1.4. ORDER OF CALCULATIONS AND RECYCLING STREAMS

It is very important to determine the best order of calculation, i.e. the order of calling each calculation subroutine by the main executive program, so that unnecessary repetition especially in the case of recycling streams to be avoided and computing time to be reduced. Lee et al. (1966) [9], Lee and Rudd (1966) [10] and Christensen and Rudd (1969) [11], give an extended literature on this subject. Kehat and Shachan (1973) [12] compare different techniques to determine the best order of calculations.

After determination of the order of calculation, processing units are taken sequentially until a recycle stream is encountered. Then the loop is torn and calculation is carried out iteratively until satisfactory convergence is obtained. The program then reverts to sequential calculation, until a new recycle stream is encountered. Two main criteria for convergence are used, as described by Cavett (1963) [13] and Kenny (1966) [14], for determining if the recycle streams have converged to a final solution:

| Mr,k   - Mr,k+1 |  <  e                                            (1.2)

| Mr,k   - Mr,k+1 |

————————   <  e                                          (1.3)

| Mr,k+1 |

where: Mr,k , Mr,k+1 = total output flow rates from unit r

k = number of iteration

e = desirable closure tolerance

Closure tolerance is most frequently, set by the program but in some cases it is set by the user. There are some troubles existing with both closure criteria [eq. (1.2) and eq. (1.3)]. Eq. (1.2) is highly dependent on small differences between large numbers, while eq. (1.3) is highly sensitive to small numbers. In either case, minor fluctuations in the controlling variable can cause unneeded iterations.

Several methods of recycle acceleration calculations have been proposed, except that of direct substitution (i.e. the use of last calculated value Μr as the assumption for the next iteration). Cavett (1963) [13] describes some recycle acceleration methods. One of the most important acceleration methods is that of Wegstein (1958) [15]. Fig. (1.4) shows a typical convergence using direct substitution method and a recycle acceleration method.

Fig.(1.4). Typical diagram for convergence of recycle stream flow calculations.

(a) Method of direct substitution.

(b) Method of recycle stream acceleration.

1.2. METHOD OF SYSTEM OF ALGEBRAIC EQUATIONS

In this method the problem to be solved is the development of a procedure to solve a system of algebraic equations defined by eq. (1.1). The major advantage of this method is the selection of all decision variables by the user, which do not necessarily coincide with a group of variables selected by a program developer, who uses the method of different calculation subroutines. On the other hand, the major disadvantage of this method is the use of complicated computer programs, which requires large computing time.

Lee et al. (1966) [9] and Edie and Westerburg (1971) [16] presented several criteria for the selection of decision variables and algorithms to solve the system of algebraic equations. Numerical methods to solve the system of algebraic equations have been also proposed. {Rosen (1962) [17], Orbach (1969) [18]}. There are also references presenting some other methods {Cavett (1963) [13]}.

For a system of algebraic equations defining the mass balances of a processing system, split fractions principle is usually used, which is based on the recycling theory of Nagiev (1964) [19] and is described by Henley and Rosen (1969) [20]. For a processing system having n processing units, there will be a system of n mass balance equations for each component c. These equations are written in the form of matrix as shown in Fig. (1.5). For example, process flow diagram of Fig. (1.2) was transformed into information block flow diagram and all equation defining the system, are presented in Fig. (1.6). Some changes it was necessary to be done in the information block flow diagram, to enable calculations. An imaginary stream G3,c, which represents acetone and hydrogen, was necessary to be added as input stream to the reactor (block 3), where the following chemical reaction takes place:

C3 H7 OH ———► CH3 ―CO―CH3  + H2

Acetone and hydrogen it is not possible to be presented as split fractions in the input stream to the reactor, as they are products of the reaction itself, but their presence as imaginary input stream helps the completion of the system of equations. Also block 7 was added as dummy unit, in order to enable final products calculations, since by this method only the input streams to every unit are calculated. Water recycling stream was presented as input stream to the processing system in the water scrubber unit and as product of the system from isopropanol column, to enable calculations for estimation of required water quantity.

Practically most split fractions are zero and so most of matrix elements are zero. Methods of matrices inversion are given by Willoughby (1969) [21]. By this way the system of equations will be solved for each one of its elements.

It is obvious that user has to do a lot of work to prepare the required by the computer program data, which is not required by the other method of different calculation subroutines.

|  | 1-f1,1,c | -f1,2,c | -f1,3,c | . | . | . | . | -f1,n,c |  |  | M1,c |  |  |  | G1,c |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -f2,1,c | 1-f2,2,c | -f2,3,c | . | . | . | . | -f2,n,c |  |  | M2,c |  |  |  | G2,c |  |
|  | -f3,1,c | -f3,2,c | 1-f3,3,c | . | . | . | . | -f3,n,c |  |  | M3,c |  |  |  | G3,c |  |
|  | . | . | . | . | . | . | . | . |  |  | . |  |  |  | . |  |
|  | . | . | . | . | . | . | . | . |  |  | . |  | = |  | . |  |
|  | . | . | . | . | . | . | . | . |  |  | . |  |  |  | . |  |
|  | . | . | . | . | . | . | . | . |  |  | . |  |  |  | . |  |
|  | -fn,1,c | -fn,2,c | -fn,3,c | . | . | . | . | 1-fn,n,c |  |  | Mn,c |  |  |  | Gn,c |  |

fj,i,c 	= Part of total flow rate of component c in the output stream from unit j and in the input stream to unit i, known as split fraction coefficient.

Mi,c	= Flow rate of component c in the input stream to unit i.

Gi,c	= Flow rate of component c in the input stream to unit i as input stream to the processing system (fresh feed).

Fig. (1.5). Mass balance equations in the form of matrices for component c, using the method of split fractions.

### A

1. Mixer

2. Steam Heater

3. Reactor

4. Water Condenser

5. Scrubber

6. Mixer

7. Acetone Column

8. Isopropanol Column

### B

|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 1 | 0 | 0 | 0 | 0 | 0 | 0 | -f1,8,c | 0 |  |  | M1,c |  |  |  | G1,c |  |
| 2 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | M2,c |  |  |  | 0 |  |
| 3 |  | 0 | -f3,2,c | 1 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | M3,c |  |  |  | G3,c |  |
| 4 |  | 0 | 0 | -f4,3,c | 1 | 0 | 0 | 0 | 0 | 0 |  |  | M4,c |  |  |  | 0 |  |
| 5 |  | 0 | 0 | 0 | -f5,4,c | 1 | 0 | 0 | 0 | 0 |  |  | M5,c |  | = |  | G5,c |  |
| 6 |  | 0 | 0 | 0 | -f6,4,c | -f6,5,c | 1 | 0 | 0 | 0 |  |  | M6,c |  |  |  | 0 |  |
| 7 |  | 0 | 0 | 0 | 0 | 0 | -f7,6,c | 1 | 0 | 0 |  |  | M7,c |  |  |  | 0 |  |
| 8 |  | 0 | 0 | 0 | 0 | 0 | 0 | -f8,7,c | 1 | 0 |  |  | M8,c |  |  |  | 0 |  |
| 9 |  | 0 | 0 | 0 | 0 | -f9,5,c | 0 | -f9,7,c | -f9,8,c | 1 |  |  | M9,c |  |  |  | 0 |  |

Fig. (1.6). Mass balance equations in the form of matrices for component c, using the method of split fractions

(a) Information block flow diagram.

(b) Mass balance equations in the form of matrices.

1.3. PHYSICAL PROPERTIES DATA

To ensure the success of computer aided flow-sheeting calculations, it is important to feed the program with a suitable data bank of physical properties, necessary for these calculations. The selection of the proper physical properties data bank is one of the most important decisions {Winter (1989) [22]}.

In most cases there is a possibility for the user to make use of the internal physical properties data bank, supplied with the program, or to feed the program with other physical properties data. In some cases the program provides the user with the possibility to decide about the method of calculation of the physical properties data.

The selection of the proper physical properties for processes where high fluctuations in pressures and temperatures occur, or in cases of processing food or agricultural products, is considered to be the most important step for the success of flow-sheeting calculations.

1.4. THE PRESENT WORK

The absence of a useful program for simulation of units processing agricultural products mainly due to lack of physical properties data for these products, was the reason to develop a flow-sheeting computer program, which provides the user with:

- The ability to perform mass and energy balances calculations, mainly for units processing agricultural products, with the least input data requirements.

- A user-friendly environment for data introduction and results presentation.

- An internal data bank for the most widely used chemical compounds and many agricultural products. User has the ability to feed the program with his own physical properties data. Especially for physical properties data concerning agricultural products, the program treats these products assuming that the majority of processed agricultural products exist either as soluble solids or insoluble solids in water, or in some cases as liquids, where no phase change takes place during processing. By this way, processes where there are changes in concentrations of soluble or suspended solids (such as evaporations, dilutions etc.) are calculated easily. These cases exist frequently in units processing agricultural products.

- Microsoft Widows environment, which enables the user to use all its conveniences.

The computer program includes the following parts:

- Main program

- Subroutines reading input data

- Subroutines arranging units and data

- Subroutines simulating production units (mixers, dividers, separators, heat exchangers, steam heaters, water coolers, evaporators, condensers, driers, phase separators, reactors etc.)

- Subroutines calculating enthalpies

- Subroutines calculating gas/liquid equilibrium

- Subroutines presenting calculation results

The selected and combined mathematical models are simulating the following types of unit operations:

- Mixer and temperature or phase set model.

- Pressure set model.

- Stream divider.

- Components separator.

- Stream divider specifying components fraction.

- Purge or make-up unit.

- Heat exchanger without phase change.

- Heat exchanger without phase change, with cooling water in the non-process side.

- Evaporator/condenser.

- Heater using steam as heating medium or a waste heat boiler producing steam.

- Evaporator using steam in the non-process side.

- Condenser using water in the non-process side.

- Single effect or one effect of a multiple-effect evaporator with specified evaporation temperature and calculated quantity of evaporated water.

- Single effect or one effect of a multiple-effect evaporator using saturated steam in the non-process side, with specified evaporation temperature and fraction of one component in the process stream.

- Drier using hot air, with specified final products temperatures and calculated quantity of evaporated water.

- Drier using hot air in the non-process stream, with specified final product humidity.

- Water cooler (cooling tower) with specified temperature and relative humidity of hot air and calculated quantity of evaporated water.

- Water cooler (cooling tower) using air in the non-process side, with specified final water temperature.

- Liquid/liquid phase separator.

- Absorber or stripper or liquid/liquid extractor or solid/liquid extractor.

- Vapour/liquid separator.

- Simple reactor.

The data bank includes the following physical properties data:

- Molecular weights (for chemical compounds).

- Melting and boiling points (for chemical compounds).

- Heats of formation, melting and evaporation (for chemical compounds).

- Critical temperatures and pressures (for chemical compounds).

- Specific heat equation constants for gas, liquid and vapour phases (for chemical compounds and agricultural products).

- Antoine vapour pressure equation constants (for chemical compounds).

An analytical presentation of this program is given in the next chapters.