5. APPLICATIONS AND RESULTS

# ACCURACY OF CALCULATIONS

The accuracy of calculations of the program was checked using applications of actual processes, existing in industries. Mass and energy balances calculations were carried out and the obtained results were compared with actual processes, existing either in industries or described in the mentioned references. Most of applications were related to process of agricultural industries, but on the other hand, some examples concerning pure chemical industries were used also in order to show the usefulness of the program for other simple cases except of agricultural industries.

In this chapter, some of the applications used will be described. These applications are:

- Calculation of adiabatic flame temperature of methane

- Water and steam requirements for a two stages tomato paste evaporator.

- Air requirements for a cooling tower of a tomato paste evaporator.

- Production of instant coffee.

- Production of olive oil.

- Flash separation of a mixture of benzene – toluene at specified conditions.

Analytical results of calculations and all required data and computer outputs are given in detail in Appendix C.

5.1. CALCULATION OF ADIABATIC FLAME TEMPERATURE OF METHANE

Adiabatic combustion of methane takes place according to the following reaction:

CH4 + 2O2 ———► CΟ2 + 2H2O

As a basis of calculations, 100 Kmoles/hr of methane is taken and consequently, the corresponding quantity of air, (i.e. 200 Kmoles/hr of oxygen and 754.37 Kmoles/hr of nitrogen), while it is considered that the whole quantity of methane is consumed. Process flow diagram is shown in Fig. (5.1).

By this example, the accuracy of trial and error adiabatic flame temperature calculations was checked, (unit 2, type 22). The calculated temperature was 2356.4°K, while the production of very small quantities of hydrogen and oxygen from the produced water and the production of very small quantities of carbon monoxide and oxygen from the produced carbon dioxide, were not taken into account. Calculated temperature is within the ranges mentioned in many references (Dodge [34]), while similar calculations of program ASPEN PLUS give 2222.2°K, using special data bank for combustion.

Fig. (5.1). Flow diagram for calculation of adiabatic flame temperature of methane.

A. Process flow diagram.

B. Block flow diagram.

1. Mixer (Type 01)

2. Reactor (Type 22)

5.2. WATER AND STEAM REQUIREMENTS FOR A TWO STAGES TOMATO PASTE EVAPORATOR

Tomato juice, having a concentration of 5°Brix, is preheated to 333°K, using steam at 378°K as the heating medium and then the a quantity of water is evaporated, until the final concentration of produced tomato paste will be 30°Brix.

A two stages evaporator is used. The temperature of evaporation in the juice entrance effect (stage) is 315°K, while the heating medium is water vapour produced in the other effect and having a temperature of 335°K.

In the other effect (stage), the temperature of evaporation is 335°K, while the heating medium is steam having a temperature of 378°K. Produced water vapour in this effect is condensed at a temperature of 313°K in a condensing tower, using water as the cooling medium with entrance temperature of 303°K and exit temperature of 313°K. Inlet temperature of cooling water (303°K) is that of exit water from a cooling tower, described in the following example 5.3. Inlet and exit temperature of condenser cooling water, have a difference of 10°K  (313°K - 303°K), instead of the usual difference of 20°K (313°K - 293°K), so that the use of a cooling tower, operating with this temperature difference to be possible. The use of a cooling tower enables the cooling water recycling, and as a result, the economy in a large quantity of water consumption. Process flow diagram is shown in Fig. (5.2).

By this example, it was possible to check the use of agricultural products data bank, and also steam requirements for the heater (unit 1, type 10) and the evaporator (unit 2, type 13 and unit 3, type 14) and water requirements for the evaporator’s condenser (unit 4, type 12). The last quantity will be used in the following example 5.3, in order to calculate the air requirements of the cooling tower.

Steam requirements was found to be 12593 Kg/hr, which is given also in many references [30] for the specific type of evaporator (Rossi & Catelli τύπος ANTEO D.F.F. T/60), while the calculated quantity of the required cooling water for the evaporator condenser, is 695599 kg/hr, instead of 300000 Kg/hr mentioned in many references [30], but for the half temperature difference (10°K instead of 20°K) mentioned in references, using fresh water, instead of recycled water from a cooling tower

Fig. (5.2). Flow diagram for calculation of water and steam requirements for a two stages tomato paste evaporator.

A. Process flow diagram.

B. Block flow diagram.

1. Steam heater (Type 10)

2. Evaporator stage (Type 13)

3. Evaporator stage (Type 14)

4. Condenser (Type 12)

5.3. AIR REQUIREMENTS FOR A COOLING TOWER OF A TOMATO PASTE EVAPORATOR

Water having a temperature of 313°K and a total quantity of 707546 Kg/hr, from a water condenser of a tomato paste evaporator, enters a cooling tower, where it is cooled to 303°K by an air stream, which enters with a temperature of 303°K and 60% relative humidity by weight and exits with a temperature of 313°K and 90% relative humidity by weight. From the cold water stream, there is a purge stream, in order to remove the extra quantity of water (from the condensation of water vapour of the exit evaporator stage, which has been added to the cooling water quantity), in order to keep the quantity of cooling water in the level of 695599Kg/hr at 303°K, required for the condenser of the previous example. Process flow diagram is shown in Fig. (5.3).

This example is considered as a continuation of the previous example 5.2. Air requirements according to the calculations (unit 2, type 18), are 348456Kg/hr at 303°K and 60% relative humidity by weight and exit the cooling tower having a quantity of 358981 Kg/hr at 313°K and 90% relative humidity by weight, carrying a quantity of 10525 Kg/hr of evaporated water, representing 1.49% of the entered water for cooling. These quantities are in agreement with the actual processing quantities [30].

Fig. (5.3). Flow diagram for calculation of air requirements for a cooling tower of a tomato paste evaporator.

A. Process flow diagram.

B. Block flow diagram.

1. Mixer (Type 01)

2. Cooling tower (Type 18)

3. Purge – Make up (Type 06)

5.4. PRODUCTION OF INSTANT COFFEE

Green coffee beans are roasted in a hot air stream at a temperature of 480°K (inlet air temperature is 500°K and humidity 0%, outlet air temperature is 420°K), where they will loose 16% of their weight and they will exit with almost 0% humidity. Then they will be cooled to a temperature of 310°K.

Roasted coffee is then grinded and its soluble solids will be extracted with hot water, heated in a steam heater at a temperature of 430°K and high enough pressure of 690KN/m², using steam at 473°K. At these conditions, almost all the quantity of soluble solids is dissolved in water and the temperature of the extract is lowered to 355°K. The extract then is cooled in a water cooler to 303°K (inlet cooling water temperature is 293°K and outlet water temperature is 343°K).

Then the extract enters a steam heated evaporator using steam at 378°K, where evaporation takes place at a temperature of 310°K. Water vapour is condensed in a condenser at a temperature of 306°K, using water as cooling medium (inlet cooling water temperature is 293°K and outlet cooling water temperature is 306°K). The extract leaves the evaporator with 40% soluble solids and enters a hot air drier (inlet air temperature is 400°K and humidity 0% and outlet air temperature is 320°K). The humidity of the final product leaving the drier is 3%. As a basis of calculations, 1000Kg/hr of green coffee beans and 1500Kg/hr of hot water are taken. Process flow diagram is shown in Fig. (5.4).

By this example, it was possible to check the use of agricultural products data bank, and also steam requirements for the evaporator, water requirements for the evaporator’s condenser and air requirements for the drier (unit 7, type 16). Obtained results are in agreement with data mentioned in references [33].

Fig. (5.4). Flow diagram for production of instant coffee.

A. Process flow diagram.

B. Block flow diagram.

1. Roaster (Type 16)

2. Air cooler (Type 01)

3. Steam heater (Type 10)

4. Extractor (Type 20)

5. Water cooler (Type 08)

6. Evaporator (Type 14)

7. Drier (Type 16)

8. Condenser (Type 12)

5.5. PRODUCTION OF OLIVE OIL

Crushed olives are mixed in a softener with water, having a temperature of 308°K. Before entering the decanter, they are mixed again with an extra quantity of water, having also a temperature of 308°K. The mixture enters a decanter, where olive stones, waste water and oil are separated.

Oil is mixed with water, having also a temperature of 308°K and enters a centrifugal separator, where pure oil is separated from waste water.

To reduce the quantity of waste and to reuse a quantity of pure water, all produced quantity of waste water is mixed and enters a centrifugal separator, where almost pure water is separated from waste. Make – up water is added and the whole quantity of water is reheated to 308°K, using steam heater, and then it is recycled.

Calculations were done on the basis of 1000 Kg/hr of crushed oil. Process flow diagram is shown in Fig. (5.5).

By this example, calculations of separation units were tested (unit 3, type 04 and units 5 & 7, type 05). Calculations results are in agreement with actual plant quantities.

Fig. (5.5). Flow diagram for production of olive oil.

A. Process flow diagram.

B. Block flow diagram.

1. Softener (Type 01)

2. Mixer (Type 01)

3. Decanter (Type 04)

4. Mixer (Type 01)

5. Centrifugal separator (Type 05)

6. Mixer (Type 01)

7. Centrifugal separator (Type 05)

8. Make – up (Type 06)

9. Steam heater (Type 10)

10. Stream divider (Type 03)

5.6. FLASH SEPARATION OF A MIXTURE OF BENZENE – TOLUENE AT SPECIFIED CONDITIONS

A mixture of benzene – toluene enters a flash separator at the following separation conditions: 75 Kmoles/hr of benzene and 25 Kmoles/hr of toluene at 366.5°K and 120KN/ m².

A total quantity of 100 Kmoles/hr of mixture was taken as a basis of calculations. It is required to calculate the compositions and quantities of the resulting two phases. Process flow diagram is shown in Fig. (5.6).

It was found that the composition of gas phase is 83.24% benzene and 16.76% toluene and the quantities are 41.78 Kmoles/hr of benzene and 8.41 Kmoles/hr of toluene. Also for the liquid phase the composition is 66.69% benzene and 33.31% toluene and the quantities are 33.22 Kmoles/hr of benzene and 16.59 Kmoles/hr of toluene. This example is a test for the accuracy of calculations for vapour/liquid flash separator (unit 1, type 21). The results are close to the mentioned in references [35].

Fig. (5.6). Flow diagram for flash separation of a mixture of benzene – toluene at specified conditions.

A. Process flow diagram.

B. Block flow diagram.

1. Flash separator (Type 21)