APPENDIX A

## DATA REQUIREMENTS

Table (A.1). Required input data.

| NAME | SIZE | DESCRIPTION |
| --- | --- | --- |
| NC NP NF NE COMP(I) PHPR(I,J) GNFD(I,J) EQPM(I,J) | NC=1 to 14 NP=1 to 20 NF=1 to 9 NE=1 to 10 I=1 to NC I=1 to NC J=1 to NP+1 (3) I=1 to NF J=1 to NC+6 I=1 to NE J=1 to NM NM=EQPM(I,3) | Number of components Number of physical properties Number of input streams Number of processing units Names of components Physical properties array(1)(2) General streams arrangement (1) Processing units array (1) |

(1) Each one of these arrays is described separately in Tables (1.2), (1.3) and (1.4)

(2) Physical properties array PHPR is usually taken from an internal physical properties data bank.

(3) Element PHPR(I,NP+1) is the same with element PHPR(I,1) and it is used by the program – after procession – to identify chemical compounds from agricultural products.

Table (A.2.a). Description of array PHPR(I,J) for chemical compounds.

Name: 	PHPR(I,J), Array of physical properties data

Range:	I=1 to NC and J=1 to NP

Description:

PHPR(I,1) =	Molecular weight of compound Ι

PHPR(I,2) =	Freezing point of compound I at 1 at abs.  [°K]

PHPR(I,3) =	Boiling point of compound I at 1 at abs.  [°K]

PHPR(I,4) =	A

PHPR(I,5) =	B

PHPR(I,6) =	C

PHPR(I,7) =	D

A,B,C,D are constants of gas phase heat capacity equation for component I

C=A+BT+CT²+DT³, where Τ is the temperature in °Κ  [J/(°K Kmole)]

PHPR(I,8) =	A

PHPR(I,9) =	B

A,B are constants of liquid phase heat capacity equation for component I

C=A+BT, where Τ is the temperature in °Κ  [J/(°K Kmole)]

PHPR(I,10)=	A

PHPR(I,11)=	B

PHPR(I,12)=	C

A,B,C are constants of solid phase heat capacity equation for component I

C=A+BT+CT², where Τ is the temperature in °Κ  [J/(°K Kmole)]

PHPR(I,13)=	Heat of melting of compound I at 1 at abs.  [J/Kmole]

PHPR(I,14)=	Heat of vaporization of compound I at 1 at abs.  [J/Kmole]

PHPR(I,15)=	Heat of formation of compound I at  298 °K and 1 at abs.  [J/Kmole]

PHPR(I,16)=	Critical temperature of compound I  [°K]

PHPR(I,17)=	Critical pressure of compound I  [N/m²]

PHPR(I,18)=	A

PHPR(I,19)=	B

PHPR(I,20)=	C

A,B,C are constants of vapour pressure Antoine equation for component I

Pvap=133.32{exp[A-B/(T+C)]}, where Τ is the temperature in °Κ  [N/m²]

N.B.1: Components in array PHPR(I,J) must be in the same order as in array COMP(I).

Table (A.2.b). Description of array PHPR(I,J) for agricultural products.

Name:  	PHPR(I,J), Array of physical properties data

Range:	I=1 to NC and J=1 to NP

Description:

PHPR(I,1) =	1

PHPR(I,2) =	0.0 for liquid phase and 999999 for solid phase

PHPR(I,3) =	999999 for solid and liquid phase

PHPR(I,4) =	0.0

PHPR(I,5) =	0.0

PHPR(I,6) =	0.0

PHPR(I,7) =	0.0

PHPR(I,8) =	A

PHPR(I,9) =	B

A,B are constants of liquid phase heat capacity equation for component I

C=A+BT, where Τ is the temperature in °Κ  [J/(°K Kg)]

PHPR(I,10)=	A

PHPR(I,11)=	B

A,B are constants of solid phase heat capacity equation for component I

C=A+BT, where Τ is the temperature in °Κ  [J/(°K Kg)]

PHPR(I,12)=	0.0

PHPR(I,13)=	0.0

PHPR(I,14)=	0.0

PHPR(I,15)=	0.0

PHPR(I,16)=	999999

PHPR(I,17)=	999999

PHPR(I,18)=	0.0

PHPR(I,19)=	0.0

PHPR(I,20)=	0.0

N.B.1: Components in array PHPR(I,J) must be in the same order as in array COMP(I).

N.B.2: Data for heat capacity equation, are given only for one phase, assuming that there is no phase change of this compound during processing. By this way, data are controlled for elements PHPR(I,2), PHPR(I,3), PHPR(I,16) and PHPR(I,17).

Table (A.3). Description of array GNFD(I,J).

Name:	GNFD(I,J), Array of streams data

Range: 	I=1 to NF and J=1 to NL , where NL=6+NC (Streams from I=1 to I=NF are feed

streams to the whole processing system)

Description:

GNFD(I,1) =	Number of feed stream to the whole processing system

0.1 for the 1st feed stream (for I=1)

0.2 for the 2nd feed stream (for I=2)

:		:

0.9 for the 9th feed stream (for I=9 maximum)

GNFD(I,2) =	Number of processing unit to which the feed stream to the whole system

enters as input stream Ι. It is a number between 1.0 and 10.0

GNFD(I,3) =	Stream phase

0.0=Undefined

1.0=Solid

2.0=Liquid

3.0=Gas

4.0=Solid/Liquid

5.0=Solid/Gas

6.0=Liquid/Gas

7.0=Solid/Liquid/Gas

GNFD(I,4) =	Stream temperature in °K or 0.0 if undefined

GNFD(I,5) =	Stream pressure in N/m² or 0.0 if undefined

GNFD(I,6) =	Stream enthalpy in J or 0.0 if undefined

GNFD(I,7) =	Quantity of 1st component in the Ith stream in Kmoles/hr or Kg/hr

GNFD(I,8) =	Quantity of 2nd component in the Ith stream in Kmoles/hr or Kg/hr

:		:

GNFD(I,NL)=	Quantity of (NC)th component in the Ith stream in Kmoles/hr or Kg/hr

N.B.1: Components in each stream must be in the same order as in array COMP(I). If a component does not exist then its quantity is considered as 0.0

N.B.2: Data of streams from I=1 to I=NF, occupy the first NF places in the array, specifying so the data of all feed streams to the whole processing system, while the rest 50-NF places will be occupied by the calculated streams data of the processing system, I.e. the maximum number of system streams are 50, including the feed streams to the system.

Table (A.4). Description of array EQPM(I,J).

Name:  EQPM(I,J), Array of processing units

Range:  I=1 to NE and J=1 to NM, where NM=EQPM(I,3)

Description:

EQPM(I,1) =	Serial number of processing unit, which is a number between 1.0 – 10.0,

defining the calculation order of the unit

EQPM(I,2) =	Type of processing unit defined by the tables of processing units

EQPM(I,3) =	Number of data required by the processing unit = NM

EQPM(I,4) =	Number of processing unit to which the 1st output stream from EQPM(I,1)

enters as input stream

EQPM(I,5) =	Number of processing unit to which the 2nd output stream from EQPM(I,1)

enters as input stream

:		:

EQPM(I,8) =	Number of processing unit to which the 6th output stream from EQPM(I,1)

enters as input stream

EQPM(I,9) =	Number of input streams to EQPM(I,1)

EQPM(I,10)=	Data required by the processing unit

EQPM(I,11)= 	Data required by the processing unit

:		:

EQPM(I,NM)= 	Data required by the processing unit

N.B.1: Units in array EQPM(I,J) from I=1 to I=10 must be in calculation order, with numbers from 1.0 to 10.0 (maximum)

N.B.2: In some cases, elements from EQPM(I,4) to EQPM(I,8) represent final product streams. In these cases:

0.1 represents the 1st product stream

0.2 represents the 2nd product stream

:		:

0.9 represents the 9th product stream (maximum)

N.B.3: In some processing units, element EQPM(I,5) and/or element EQPM(I,6) represent non-process streams. In these cases:

0.01 represents purge or make-up stream

0.02 represents cooling water

0.03 represents stream production

0.04 represents steam condensation

0.05 represents air

Table (A.5). Arrangement of required data.

| DATA | FORMAT |
| --- | --- |
| TITLE NC NP NF NE COMP(1) COMP(2) COMP(3) : : COMP(NC) PHPR(1,1)....................................PHPR(1,5) PHPR(1,6).........…………………..PHPR(1,10) : PHPR(1,21) PHPR(2,1)....................................PHPR(2,5) PHPR(2,6)........…………………...PHPR(2,10) : PHPR(1,21) : : : : PHPR(NC,1)..................................PHPR(NC,5) PHPR(NC,6).....…………………....PHPR(NC,10) : PHPR(NC,21) GNFD(1,1)....................................GNFD(1,5) GNFD(1,6)...........GNFD(1,NL) GNFD(2,1)....................................GNFD(2,5) GNFD(2,6)...........GNFD(2,NL) : : : : GNFD(NF,1)..................................GNFD(NF,5) GNFD(NF,6)..........GNFD(NF,NL) EQPM(1,1),EQPM(1,2),EQPM(1,3) EQPM(1,4).....................................EQPM(1,8) EQPM(1,9)....…....EQPM(1,NM) EQPM(2,1),EQPM(2,2),EQPM(2,3) EQPM(2,4).....................................EQPM(2,8) EQPM(2,9).....…...EQPM(2,NM) : : : : EQPM(NE,1),EQPM(NE,2),EQPM(NE,3) EQPM(NE,4)..................................EQPM(NE,8) EQPM(NE,9).........EQPM(NE,NM) | A 4I14 A A A : : A 5F14.0 5F14.0 : F14.0 5F14.0 5F14.0 : F14.0 : : 5F14.0 5F14.0 : F14.0 5F14.0 5F14.0 5F14.0 5F14.0 : : 5F14.0 5F14.0 3F14.0 5F14.0 5F14.0 3F14.0 5F14.0 5F14.0 : : 3F14.0 5F14.0 5F14.0 |

Table (A.6). Main variables and arrays of the program.

| NAME | RANGE | DESCRIPTION |
| --- | --- | --- |
| FEED(I,J) PROD(I,J) NI NO NR NL TOTMLOLD(I) TOTMLNEW(I) NCOUNTER(I) | I=1 to NI J=1 to NC+6 I=1 to NO J=1 to NC+6 NI=1 to 5 (max) NO=1 to 5 (max) NR=1 to 10 (max) NL=NC+6 NL=7 to 20 (max) I=1 to 50 I=1 to 50 I=1 to 50 | Data array of input streams to each processing unit Data array of output streams from each processing unit Number of input streams to each processing unit Number of output streams from each processing unit Current number of processing unit Number of elements of arrays GNFD, FEED and PROD Array of calculated total quantity for recycling stream I Array of calculated total quantity for recycling stream I by next iteration. [To be compared with corresponding calculated quantity in array TOTMLOLD(I)] Array of number of iterations for recycling stream I |

### Table (A.7.01). Presentation of subroutine UNIT01

__________________________________________________________________________

###### Graphical presentation of processing unit type: 01

Description:

This type of processing unit adds (i.e. mixes) up to 5 input streams. It calculates the temperature of output stream, assuming adiabatic operation, if the temperatures of all input streams is specified, or it sets the output stream temperature equal to a specified value. It sets the output stream pressure equal to the smaller pressure of the input streams, while the phase of output stream is specified. This type of subroutine is used also to specify the temperature or the phase of a single stream.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	1.0

EQPM(NR,3) =	NM=12.0

EQPM(NR,4) =	Number of processing unit to which the output stream enters as input stream

EQPM(NR,5) =	0.0

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) = Number of input streams = 1.0 or 2.0 or 3.0 or 4.0 or 5.0

EQPM(NR,10)=	Phase of output stream

EQPM(NR,11)=	1.0 for adiabatic operation or

2.0 for specified temperature of operation

EQPM(NR,12)=	Temperature of operation in °Κ [if EQPM(NR,11)=2.0] or

0.0 for adiabatic operation [if EQPM(NR,11)=1.0]

__________________________________________________________________________

### Table (A.7.02). Presentation of subroutine UNIT02

__________________________________________________________________________

###### Graphical presentation of processing unit type: 02

Description:

This type of processing unit sets the pressure of a single stream equal to a specified value, without change in phase, temperature or composition.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	2.0

EQPM(NR,3) =	NM=10.0

EQPM(NR,4) =	Number of processing unit to which the output stream enters as input stream

EQPM(NR,5) =	0.0

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Pressure of output stream in KN/m²

__________________________________________________________________________

### Table (A.7.03). Presentation of subroutine UNIT03

__________________________________________________________________________

###### Graphical presentation of processing unit type: 03

Description:

This type of processing unit represents a stream splitter, which splits a single input stream in up to 5 output streams, without change in phase, temperature, pressure or composition. The fractions of total input stream flow rate, consisting each output stream flow rate, are specified.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	3.0

EQPM(NR,3) =	NM=9.0+NO

EQPM(NR,4) =	Number of processing unit to which the 1st output stream enters as input

stream

EQPM(NR,5) = Number of processing unit to which the 2nd output stream enters as input

stream

:		:

EQPM(NR,8) = Number of processing unit to which the 5th output stream enters as input

stream

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Fraction of total input consisting the 1st output stream

EQPM(NR,11)= Fraction of total input consisting the 2nd output stream

:		:

EQPM(NR,NO+9)= Fraction of total input consisting the (NO)th output stream

__________________________________________________________________________

### Table (A.7.04). Presentation of subroutine UNIT04

__________________________________________________________________________

###### Graphical presentation of processing unit type: 04

Description:

This type of processing unit represents a simple component separation unit. A single input stream is divided in up to 5 output streams, each one of them has the same temperature and pressure as the input stream, while the distribution of each component in each output stream is specified as fraction of its total input quantity. The phase of each output stream is specified.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	4.0

EQPM(NR,3) =	NM=9.0+NO*(NC+1)

EQPM(NR,4) =	Number of processing unit to which the 1st output stream enters as input

stream

EQPM(NR,5) = Number of processing unit to which the 2nd output stream enters as input

stream

:		:

EQPM(NR,8) = Number of processing unit to which the 5th output stream enters as input

stream

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Fraction of total input of 1st component in the 1st output stream

EQPM(NR,11)= Fraction of total input of 2ndcomponent in the 1st output stream

:		:

EQPM(NR,9+NC)= Fraction of total input of (NC)th component in the 1st output stream

EQPM(NR,10+NC)= Fraction of total input of 1st component in the 2nd output stream

EQPM(NR,11+NC)= Fraction of total input of 2ndcomponent in the 2nd output stream

:		:

EQPM(NR,9+2*NC)= Fraction of total input of (NC)th component in the 2nd output stream

:		:

EQPM(NR,9+NO*NC)= Fraction of total input of (NC)th component in the(NO)th output stream

EQPM(NR,10+NO*NC)= Phase of 1st output stream

EQPM(NR,11+NO*NC)= Phase of 2nd output stream

:		:

EQPM(NR,9+NO*(NC+1))= Phase of (NO)th output stream

__________________________________________________________________________

### Table (A.7.05). Presentation of subroutine UNIT05

__________________________________________________________________________

###### Graphical presentation of processing unit type: 05

Description:

This type of processing unit represents a simple separation unit. A single input stream is divided into 2 output streams, and each one of them has the same temperature and pressure as the input stream, but the presence of each component in one output stream is specified as fraction of the total flow rate of the stream, and the presence of one component in the other output stream, as fraction of the total flow rate of the stream, is specified. Phases of both output streams are specified separately.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	5.0

EQPM(NR,3) =	NM=13.0+NC

EQPM(NR,4) =	Number of processing unit to which the 1st output stream enters as input

stream

EQPM(NR,5) = Number of processing unit to which the 2nd output stream enters as input

stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Fraction of total output of 1st output stream consisting the 1st component

EQPM(NR,11)=	Fraction of total output of 1st output stream consisting the 2nd component		:		:

EQPM(NR,9+NC)= Fraction of total output of 1st output stream consisting the (NC)th

component

EQPM(NR,10+NC)= Fraction of total put of one component in the 2nd output stream

EQPM(NR,11+NC)= Ascending number of component specified in EQPM(NR,11+NC), in

array COMP(I)

EQPM(NR,12+NC)= Phase of 1st output stream

EQPM(NR,13+NC)= Phase of 2nd output stream

__________________________________________________________________________

### Table (A.7.06). Presentation of subroutine UNIT06

__________________________________________________________________________

###### Graphical presentation of processing unit type: 06

Description:

This type of processing unit represents a purge or make-up stream, with specified total flow rate of the main stream, or flow rate of one component in the main stream, or one component as fraction of the main stream.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	6.0

EQPM(NR,3) =	NM=12.0

EQPM(NR,4) =	Number of processing unit to which the main output stream enters as input

stream

EQPM(NR,5) =	Purge or make-up stream (non-process stream) = 0.01

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Total quantity of the main output stream or

quantity of one component in the main output stream or

fraction total quantity of the main output stream consisting one component

EQPM(NR,11)=	1.0 if total quantity of the main output stream is specified or

2.0 if the quantity of one component in the main output stream is specified or

3.0 if the fraction of the main output stream consisting one component is

specified

EQPM(NR,12)=	Ascending number of component specified in EQPM(NR,10), in array

COMP(I) [if EQPM(NR,11)=2.0 or EQPM(NR,11)=3.0] or

0.0 if total quantity of the main output stream is specified

[if EQPM(NR,11)=1.0]

__________________________________________________________________________

### Table (A.7.07). Presentation of subroutine UNIT07

__________________________________________________________________________

###### Graphical presentation of processing unit type: 07

Description:

This type of processing unit represents a heat exchanger, where the temperature of one output stream is specified, while the temperature of the other output stream is calculated. Phases of both input streams remain the same.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	7.0

EQPM(NR,3) =	NM=11.0

EQPM(NR,4) =	Number of processing unit to which the 1st output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the 2nd output stream enters as input

stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the 1st output stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Temperature of 1st output stream in °K

__________________________________________________________________________

### Table (A.7.08). Presentation of subroutine UNIT08

__________________________________________________________________________

###### Graphical presentation of processing unit type: 08

Description:

This type of processing unit represents a water cooler, using water as the cooling medium with specified input temperature. Output stream temperature of the process is specified, while its phase remains the same. Cooling water is not present as a process stream, but its output temperature is specified, while its flow rate is calculated.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	8.0

EQPM(NR,3) =	NM=13.0

EQPM(NR,4) =	Number of processing unit to which the process output  stream enters as

input  stream

EQPM(NR,5) =	Cooling water (non-process stream) = 0.02

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of process output stream in °K

EQPM(NR,11)=	Temperature of cooling water output stream in °K

EQPM(NR,12)=	Temperature of cooling water input stream in °K

EQPM(NR,13)=	Ascending number of water in array COMP(I)

__________________________________________________________________________

### Table (A.7.09). Presentation of subroutine UNIT09

__________________________________________________________________________

###### Graphical presentation of processing unit type: 09

Description:

This type of processing unit represents a vaporizer or condenser with specified temperature and pressure of vaporization or condensation, while the temperature of the other output stream, i.e. of the heating or cooling medium, whose the phase remains the same, is calculated. Heat of vaporization or condensation is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	9.0

EQPM(NR,3) =	NM=16.0+NMI+NIM+NNC [see EQPM(NR,14),EQPM(NR,15),EQPM(NR,16)]

EQPM(NR,4) =	Number of processing unit to which the gas output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the liquid output stream enters as input

stream

EQPM(NR,6) =	Number of processing unit to which the heating or cooling medium output

stream enters as input stream

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the process stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Temperature of vaporization or condensation in °K

EQPM(NR,12)=	Pressure of vaporization or condensation in KN/m²

EQPM(NR,13)=	1.0 if vaporization takes place

2.0 if condensation takes place

EQPM(NR,14)=	NMI=number of miscible liquids (NMI=0.0 or NMI>=2.0)

EQPM(NR,15)=	NIM=number of immiscible liquids (NIM=0.0 or NIM=1.0 if NMI>=2.0 or

NIM=2.0 if NMI=0.0 or NIM=1 if only one liquid component exists)

EQPM(NR,16)=	NNC=number of non-condensable gases

EQPM(NR,17)=Ascending number of 1st miscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI)= Ascending number of (NMI)th miscible liquid in COMP(I)

EQPM(NR,17+NMI)= Ascending number of 1st immiscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI+NIM)= Ascending number of (NIM)th immiscible liquid in COMP(I)

EQPM(NR,17+NMI+NIM)= Ascending number of 1st non-condensable gas in COMP(I)

:		:

EQPM(NR,16+NMI+NIM+NNC)= Ascending number of (NNC)th non-condensable gas in

COMP(I)

__________________________________________________________________________

### Table (A.7.10). Presentation of subroutine UNIT10

__________________________________________________________________________

###### Graphical presentation of processing unit type: 10

Description:

This type of processing unit represents a heater using saturated steam as the heating medium, or a waste heat steam boiler. The temperature of process stream is specified, while its phase remains the same. Saturated steam, which will be condensed, or water for saturated steam production, are not present as process streams, but the temperature of produced or condensed saturated steam is specified, while its flow rate is calculated. For the case of saturated steam production, the input non-process stream water temperature is specified. Heat of evaporation or condensation of water, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	10.0

EQPM(NR,3) =	NM=13.0

EQPM(NR,4) =	Number of processing unit to which the process output  stream enters as

input  stream

EQPM(NR,5) =	Produced saturated steam (non-process stream) = 0.03 or

Required saturated steam (non-process stream) = 0.04

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of process output stream in °K

EQPM(NR,11)=	Temperature of saturated steam produced or condensed in °K

EQPM(NR,12)=	Temperature of input water stream for steam production in °K

[if EQPM(NR,5)=0.03] or

0.0 [if EQPM(NR,5)=0.04]

EQPM(NR,13)=	Ascending number of water in array COMP(I)

__________________________________________________________________________

### Table (A.7.11). Presentation of subroutine UNIT11

__________________________________________________________________________

###### Graphical presentation of processing unit type: 11

Description:

This type of processing unit represents a vaporizer using saturated steam as heating medium with specified temperature. Temperature and pressure of vaporized product is specified. Saturated steam is not present as a process stream, but its flow rate is calculated. Heats of vaporization and condensation of product and steam respectively, are calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	11.0

EQPM(NR,3) =	NM=16.0+NMI+NIM+NNC [see EQPM(NR,14),EQPM(NR,15),EQPM(NR,16)]

EQPM(NR,4) =	Number of processing unit to which the gas output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the liquid output stream enters as input

stream

EQPM(NR,6) =	Required saturated steam (non-process stream) = 0.04

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of vaporization in °K

EQPM(NR,11)=	Pressure of vaporization in KN/m²

EQPM(NR,12)=	Temperature of used saturated steam in °K

EQPM(NR,13)= Ascending number of water in array COMP(I)

EQPM(NR,14)=	NMI=number of miscible liquids (NMI=0.0 or NMI>=2.0)

EQPM(NR,15)=	NIM=number of immiscible liquids (NIM=0.0 or NIM=1.0 if NMI>=2.0 or

NIM=2.0 if NMI=0.0 or NIM=1 if only one liquid component exists)

EQPM(NR,16)=	NNC=number of non-condensable gases

EQPM(NR,17)=Ascending number of 1st miscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI)= Ascending number of (NMI)th miscible liquid in COMP(I)

EQPM(NR,17+NMI)= Ascending number of 1st immiscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI+NIM)= Ascending number of (NIM)th immiscible liquid in COMP(I)

EQPM(NR,17+NMI+NIM)= Ascending number of 1st non-condensable gas in COMP(I)

:		:

EQPM(NR,16+NMI+NIM+NNC)= Ascending number of (NNC)th non-condensable gas in

COMP(I)

__________________________________________________________________________

### Table (A.7.12). Presentation of subroutine UNIT12

__________________________________________________________________________

###### Graphical presentation of processing unit type: 12

Description:

This type of processing unit represents a condenser, using water as the cooling medium, with specified input temperature. Temperature and pressure of condensed product is specified. Cooling water is not present as a process stream, but its output temperature is specified, while its flow rate is calculated. Heat of condensation of product, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	12.0

EQPM(NR,3) =	NM=17.0+NMI+NIM+NNC [see EQPM(NR,15),EQPM(NR,16),EQPM(NR,17)]

EQPM(NR,4) =	Number of processing unit to which the gas output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the liquid output stream enters as input

stream

EQPM(NR,6) =	Cooling water (non-process stream) = 0.02

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of condensation in °K

EQPM(NR,11)=	Pressure of condensation in KN/m²

EQPM(NR,12)=	Temperature of cooling water output stream in °K

EQPM(NR,13)=	Temperature of cooling water input stream in °K

EQPM(NR,14)=	Ascending number of water in array COMP(I)

EQPM(NR,15)=	NMI=number of miscible liquids (NMI=0.0 or NMI>=2.0)

EQPM(NR,16)=	NIM=number of immiscible liquids (NIM=0.0 or NIM=1.0 if NMI>=2.0 or

NIM=2.0 if NMI=0.0 or NIM=1 if only one liquid component exists)

EQPM(NR,17)=	NNC=number of non-condensable gases

EQPM(NR,18)=Ascending number of 1st miscible liquid in COMP(I)

:		:

EQPM(NR,17+NMI)= Ascending number of (NMI)th miscible liquid in COMP(I)

EQPM(NR,18+NMI)= Ascending number of 1st immiscible liquid in COMP(I)

:		:

EQPM(NR,17+NMI+NIM)= Ascending number of (NIM)th immiscible liquid in COMP(I)

EQPM(NR,18+NMI+NIM)= Ascending number of 1st non-condensable gas in COMP(I)

:		:

EQPM(NR,17+NMI+NIM+NNC)= Ascending number of (NNC)th non-condensable gas in

COMP(I)

__________________________________________________________________________

### Table (A.7.13). Presentation of subroutine UNIT13

__________________________________________________________________________

###### Graphical presentation of processing unit type: 13

Description:

This type of processing unit represents a single stage evaporator or one stage of a multistage evaporator. Temperature of water evaporation is specified, while the quantity of evaporated water is calculated and considered as the second product output stream. Heats of evaporation and condensation of water, are calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	13.0

EQPM(NR,3) =	NM=13.0

EQPM(NR,4) =	Number of processing unit to which the product output  stream enters as

input  stream

EQPM(NR,5) =	Number of processing unit to which the evaporated water stream enters as

input stream

EQPM(NR,6) =	Number of processing unit to which the condensed water vapour stream

enters as input  stream

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the product stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Temperature of water evaporation stage in °K

EQPM(NR,12)=	Temperature of used water vapours for evaporation in °K

EQPM(NR,13)=	Ascending number of water in array COMP(I)

__________________________________________________________________________

### Table (A.7.14). Presentation of subroutine UNIT14

__________________________________________________________________________

###### Graphical presentation of processing unit type: 14

Description:

This type of processing unit represents a single stage evaporator or one stage of a multi stage evaporator. Temperature of water evaporation is specified. The required quantity of one component in the main process output stream is specified as fraction of its total stream flow rate. Saturated steam required as heating medium, is not present as process stream, but its quantity is calculated. Heats of evaporation and condensation of water are calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	14.0

EQPM(NR,3) =	NM=14.0

EQPM(NR,4) =	Number of processing unit to which the product output  stream enters as

input  stream

EQPM(NR,5) =	Number of processing unit to which the evaporated water stream enters as

input stream

EQPM(NR,6) =	Required saturated steam (non-process stream) = 0.04

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of water evaporation stage in °K

EQPM(NR,11)=	Fraction of total output of product output stream consisting one component

EQPM(NR,12)=	Temperature of used water vapours for evaporation in °K

EQPM(NR,13)=	Ascending number of water in array COMP(I)

EQPM(NR,14)=	Ascending number of component specified in EQPM(NR,11), in array

COMP(I)

__________________________________________________________________________

### Table (A.7.15). Presentation of subroutine UNIT15

__________________________________________________________________________

###### Graphical presentation of processing unit type: 15

Description:

This type of processing unit represents a drier, operating with hot air, with specified input temperatures of product and air, while the quantity of evaporated water, carried by the cold air stream, is calculated, assuming adiabatic operation at 1atm abs. Heat of evaporation of water, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	15.0

EQPM(NR,3) =	NM=13.0

EQPM(NR,4) =	Number of processing unit to which the product output  stream enters as

input  stream

EQPM(NR,5) =	Number of processing unit to which the cold air output  stream enters as

input  stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the product stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Temperature of product output steam in °K

EQPM(NR,12)=	Temperature of cold air output steam in °K

EQPM(NR,13)=	Ascending number of water in array COMP(I)

__________________________________________________________________________

### Table (A.7.16). Presentation of subroutine UNIT16

__________________________________________________________________________

###### Graphical presentation of processing unit type: 16

Description:

This type of processing unit represents a drier, operating with hot air, with specified final product humidity. Hot air is not present as process stream, but its input and output temperatures are specified, while its quantity is calculated, assuming adiabatic operation at 1atm abs. Heat of evaporation of water, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	16.0

EQPM(NR,3) =	NM=17.0

EQPM(NR,4) =	Number of processing unit to which the process output  stream enters as

input  stream

EQPM(NR,5) =	Required air (non-process stream) = 0.05

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Humidity of process output stream expressed as [(quantity of water)/

(total quantity of process stream)]

EQPM(NR,11)=	Temperature of process output stream in °K

EQPM(NR,12)= Humidity of hot air input stream expressed as [(quantity of water)/

(total quantity of hot air stream)]

EQPM(NR,13)=	Temperature of hot air input stream in °K

EQPM(NR,14)=	Temperature of cold air output stream in °K

EQPM(NR,15)=	Ascending number of water in array COMP(I)

EQPM(NR,16)=	Ascending number of oxygen in array COMP(I)

EQPM(NR,17)=	Ascending number of nitrogen in array COMP(I)

__________________________________________________________________________

### Table (A.7.17). Presentation of subroutine UNIT17

__________________________________________________________________________

###### Graphical presentation of processing unit type: 17

Description:

This type of processing unit represents a water cooling system with specified temperature and relative humidity of hot air output stream, while the quantity of evaporated water carried by the hot air output stream and the cold water output stream temperature are calculated assuming adiabatic operation at 1 atm abs. Heat of evaporation of water, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	17.0

EQPM(NR,3) =	NM=15.0

EQPM(NR,4) =	Number of processing unit to which the cold water output  stream enters as

input  stream

EQPM(NR,5) =	Number of processing unit to which the hot air output  stream enters as

input  stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the hot water stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Relative humidity of hot air output stream

EQPM(NR,12)=	Temperature of hot air output steam in °K

EQPM(NR,13)=	Ascending number of water in array COMP(I)

EQPM(NR,14)=	Ascending number of oxygen in array COMP(I)

EQPM(NR,15)=	Ascending number of nitrogen in array COMP(I)

__________________________________________________________________________

# Table (A.7.18). Presentation of subroutine UNIT18

__________________________________________________________________________

###### Graphical presentation of processing unit type: 18

Description:

This type of processing unit represents a water cooling system with specified final temperature of cold water. Air is not present as process stream, but temperatures and relative humidities of cold air input stream and hot air output stream are specified, while its quantity is calculated assuming adiabatic operation at 1 atm abs. Heat of evaporation of water, is calculated using Watson equation.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	18.0

EQPM(NR,3) =	NM=17.0

EQPM(NR,4) =	Number of processing unit to which the cold water output  stream enters as

input  stream

EQPM(NR,5) =	Required air (non-process stream) = 0.05

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Temperature of cold water output stream in °K

EQPM(NR,11)=	Relative humidity of cold air input stream

EQPM(NR,12)=	Relative humidity of hot air output stream

EQPM(NR,13)=	Temperature of cold air input stream in °K

EQPM(NR,14)=	Temperature of hot air output stream in °K

EQPM(NR,15)=	Ascending number of water in array COMP(I)

EQPM(NR,16)=	Ascending number of oxygen in array COMP(I)

EQPM(NR,17)=	Ascending number of nitrogen in array COMP(I)

__________________________________________________________________________

### Table (A.7.19). Presentation of subroutine UNIT19

__________________________________________________________________________

###### Graphical presentation of processing unit type: 19

Description:

This type of processing unit represents a simple light and heavy phase separation liquid/liquid system, operating at constant temperature and constant pressure. Separation factors are considered to be constants.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	19.0

EQPM(NR,3) =	NM=12.0+NH [see EQPM(NR,12)]

EQPM(NR,4) =	Number of processing unit to which the light phase output stream enters as

input stream

EQPM(NR,5) = Number of processing unit to which the heavy phase output stream enters as

input stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Fraction of total output of heavy phase output stream consisting the light

phase

EQPM(NR,11)=	Fraction of total output of light phase output stream consisting the heavy

phase

EQPM(NR,12)=	NH=Number of components consisting the heavy phase

EQPM(NR,13)= Ascending number of 1st component consisting the heavy phase, in

array COMP(I)

EQPM(NR,14)= Ascending number of 2nd component consisting the heavy phase, in

array COMP(I)

:		:

EQPM(NR,12+NH)= Ascending number of (NH)th component consisting the heavy phase, in

array COMP(I)

__________________________________________________________________________

### Table (A.7.20). Presentation of subroutine UNIT20

__________________________________________________________________________

###### Graphical presentation of processing unit type: 20

Description:

This type of processing unit represents an absorber or stripper or liquid/liquid extractor or solid/liquid extractor, operating at constant temperature and constant pressure. The recovery of each component existing in one output stream, as fraction of its total input in the proper input stream, is specified.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	20.0

EQPM(NR,3) =	NM=13.0+NC

EQPM(NR,4) =	Number of processing unit to which the 1st output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the 2nd output stream enters as input

stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 2.0

EQPM(NR,10)=	Number of processing unit from which the 1st output stream is an output

stream before entering the current processing unit

EQPM(NR,11)=	Temperature of operation in °K

EQPM(NR,12)=	Pressure of operation in KN/m²

EQPM(NR,13)=	1.0 if the specified fractions of components concern the 1st output stream or

2.0 if the specified fractions of components concern the 2nd output stream

EQPM(NR,14)=	Fraction of total quantity of 1st component in one input stream, existing in the

output stream specified in EQPM(NR,13)

EQPM(NR,15)=	Fraction of total quantity of 2nd component in one input stream, existing in the

output stream specified in EQPM(NR,13)

:		:

EQPM(NR,13+NC)=Fraction of total quantity of (NC)th component in one input stream,

existing in the output stream specified in EQPM(NR,13)

__________________________________________________________________________

Table (A.7.21). Presentation of subroutine UNIT21

__________________________________________________________________________

###### Graphical presentation of processing unit type: 21

Description:

This type of processing unit represents a vapour/liquid separation system with specified pressure, and it calculates the output streams temperature assuming adiabatic operation, or it sets the output streams temperature assuming isothermal operation, or it sets the output streams temperature equal to a specified value.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	21.0

EQPM(NR,3) =	NM=16.0+NMI+NIM+NNC [see EQPM(NR,14),EQPM(NR,15),EQPM(NR,16)]

EQPM(NR,4) =	Number of processing unit to which the gas output stream enters as input

stream

EQPM(NR,5) =	Number of processing unit to which the liquid output stream enters as input

stream

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	1.0 for adiabatic operation or

2.0 for isothermal operation or

3.0 for specified temperature of operation

EQPM(NR,11)=	Temperature of operation in °K [if EQPM(NR,10)=3.0]or

0.0 for adiabatic or isothermal operation [if EQPM(NR,10)=1.0 or EQPM(NR,10)=2.0]

EQPM(NR,12)=	Pressure of operation in KN/m²

EQPM(NR,13)=	1.0 if vaporization takes place

2.0 if condensation takes place

EQPM(NR,14)=	NMI=number of miscible liquids (NMI=0.0 or NMI>=2.0)

EQPM(NR,15)=	NIM=number of immiscible liquids (NIM=0.0 or NIM=1.0 if NMI>=2.0 or

NIM=2.0 if NMI=0.0 or NIM=1 if only one liquid component exists)

EQPM(NR,16)=	NNC=number of non-condensable gases

EQPM(NR,17)=Ascending number of 1st miscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI)= Ascending number of (NMI)th miscible liquid in COMP(I)

EQPM(NR,17+NMI)= Ascending number of 1st immiscible liquid in COMP(I)

:		:

EQPM(NR,16+NMI+NIM)= Ascending number of (NIM)th immiscible liquid in COMP(I)

EQPM(NR,17+NMI+NIM)= Ascending number of 1st non-condensable gas in COMP(I)

:		:

EQPM(NR,16+NMI+NIM+NNC)= Ascending number of (NNC)th non-condensable gas in

COMP(I)

__________________________________________________________________________

### Table (A.7.22). Presentation of subroutine UNIT22

__________________________________________________________________________

###### Graphical presentation of processing unit type: 22

Description:

This type of processing unit represents a simple reactor for steady transformation and if the temperature of the input stream is specified, it calculates the output stream temperature assuming adiabatic operation, or it sets the output stream temperature assuming isothermal operation, or it sets the output stream temperature equal to a specified value. Output stream pressure is equal to the input stream pressure. Reaction is considered to take place at 1 atm abs. Heat of reaction is calculated from heats of generation of the reaction components, while the conversion factor of a key component is specified.

Required data:

EQPM(NR,1) =	Calculation order of processing unit

EQPM(NR,2) =	22.0

EQPM(NR,3) =	NM=14.0+NC

EQPM(NR,4) =	Number of processing unit to which the output stream enters as input stream

EQPM(NR,5) =	0.0

EQPM(NR,6) =	0.0

EQPM(NR,7) =	0.0

EQPM(NR,8) =	0.0

EQPM(NR,9) =	Number of input streams = 1.0

EQPM(NR,10)=	Phase of output stream

EQPM(NR,11)=	1.0 for adiabatic operation or

2.0 for isothermal operation or

3.0 for specified temperature of operation

EQPM(NR,12)=	Temperature of operation in °Κ [if EQPM(NR,11)=3.0] or

0.0 for adiabatic or isothermal operation [if EQPM(NR,11)=1.0 or EQPM(NR,11)=2.0]

EQPM(NR,13)=	Conversion factor of a key component in reaction

EQPM(NR,14)=	Ascending number of component specified in EQPM(NR,13), in array

COMP(I)

EQPM(NR,15)=	Stoichiometric factor for transformation of 1st component in reaction

positive if produced or

negative if consumed or

0.0 if does not take part in reaction

:		:

EQPM(NR,14+NC)=Stoichiometric factor for transformation of (NC)th component in reaction

positive if produced or

negative is consumed or

0.0 if does not take part in reaction

__________________________________________________________________________